# 7O

# TRAFFIC CONTROL AND PROTECTION, STANDARDS 701400 (SPECIAL) and 701401 (SPECIAL)

(Effective August 7, 2008; Revised March 31, 2014)

Description.  Replace any reference to Standard 701400 in Standard 701401 with the detail, Standard 701400 (Special), shown on the plans.

Construction Requirements.  This work shall be done according to the applicable portions of Section 701 and 780 of the Standard Specifications, the details on the plans, and Standards 701401 and 701901.

Temporary rumble strips, when shown on the detail, shall be constructed of white thermoplastic or temporary tape pavement markings placed to a thickness of one-half inch.

Method of Measurement.  Traffic control and protection required under the detail, Standard 701400 (Special), will not be measured for payment.

Temporary thermoplastic rumble strips and lane drop arrows, when shown on the detail, will not be measured for payment.

In Article 701.19(c) Measured As Lump Sum of the Standard Specifications, replace Standard 701401 with Standard 701401 (Special).

Basis of Payment.  This work will be paid for at the contract lump sum price for TRAFFIC CONTROL AND PROTECTION, STANDARD 701401 (SPECIAL).

DESIGNER NOTES:

If using a Standard other than 701401, replace standard number and revise pay item accordingly in special above.

Include CADD Detail 701-10, Standard 701400 (Special), Option 1 or Option 2. The Bureau of Construction will determine which option to use.

Include Standards 701401 (or other appropriate standard) and 701901 in the plans.

Include lump sum pay item X7010805 Traffic Control and Protection, Standard 701401 (Special).

To be used on all interstate contracts and when requested by Construction.  Also to be used on other routes which have an ADT >25,000.