All District Engineers, Carl Kowalski & Thomas R. Walker

William T. Sunley

Special Provision for English Substitution of Metric Reinforcement

April 16, 1997

This special provision has been revised in conjunction with the Bureau of Bridges and Structures and the Bureau of Materials and Physical Research.  Industry appears to be producing metric reinforcement only in name.  The bars will be stamped with English sizes or with soft converted metric sizes, which means the same bar can have either stamp.  The English number #5 and the soft metric #16 will be the same bar just with a different name.

The subject revised special provision will allow the Contractor to substitute English size reinforcement bars or the soft converted metric size bars for the metric size reinforcement bars shown on the plans.

In general, for structures other than slab bridges or slabs of culverts, a metric reinforcement bar shown on the plans may be replaced bar for bar with the next size English reinforcement bar or soft converted metric bar of equal or greater cross-sectional area.  The substitution must be made bar for bar so as not to change the original bar spacing.  The exception is the #5 English bar or the number #16 soft converted metric bar which may be substituted bar for bar for the #15 metric bar.

For pavements and appurtenances the metric and English sizes are shown on the Highway Standard Drawings.  The English size bar shown may be used at the metric spacing and length.  For example, a Standard Drawing shows a No. 20 (No. 6) bar 750 mm (30 inches) in length at 750 mm (30 inch) spacing.  The substitution would be a No. 6 bar, 750 mm in length, at 750 mm spacing.  The soft converted metric bar may also be substituted exactly like the English bar is substituted.  The exception to this is for the continuously reinforced pavements.  Continuously reinforced pavements have been redesigned to use  English sized bars or the soft converted metric size bars in the metric job and still have the 0.7% area of steel required.  The table in the special provision shows the allowable substitution.

All District Engineers, Carl Kowalski & Thomas R. Walker

Page 2

April 16, 1997

This special provision should be included in all metric projects containing reinforcement bars.

The districts should include the BDE Check Sheet with the applicable BDE Special Provisions marked for the August 1, 1997 letting and for subsequent lettings and the Project Development and Implementation Section will include the paper copy in the contract.

This special provision will be transferred through the Wang Office system to the district offices on April 25, 1997.

CK:sam\/tmp/in/input.doc

English Substitution of Metric Reinforcement Bars (BDE)

Effective:  April 1,1996

Revised:  July 15, 1997

This special provision consists of giving the Contractor the option of replacing metric reinforcement bars as shown on the plans with English size reinforcement bars or metric size bars which have been soft converted.  Soft Conversion is an exact conversion to the nearest millimeter.

Reinforcement for Structures:

A metric reinforcement bar shown on the plans may be replaced bar for bar with the next size English bar or soft converted metric reinforcement bar of equal or greater cross-sectional area.  The exception is the #5 English bar or #16 soft converted metric bar may be substituted bar for bar for the #15 metric bar shown on the plans.

For slab bridges and slabs of culverts, revaluation of the slab design will be required prior to any reinforcement bar substitutions.  The Contractor shall submit the design to the Bridge Office for approval.

| Metric Size Shown on the Plans | Area mm2 |  | English Size | Metric Size Soft Converted (ASTM A 615 m - 96a) | Area mm2 |
| --- | --- | --- | --- | --- | --- |
| #10 | 100 |  | #4 | #13 | 127 |
| #15 | 200 |  | #5 | #16 | 198 |
| #20 | 300 |  | #7 | #22 | 388 |
| #25 | 500 |  | #8 | #25 | 507 |
| #30 | 700 |  | #10 | #32 | 817 |
| #35 | 1000 |  | #11 | #36 | 1007 |
| #45 | 1500 |  | #18 | #57 | 2581 |

Reinforcement for Pavement:

For English substitution of metric bars in pavements and appurtenances the Contractor may use the given English sizes shown on the Standards or metric size bars which have been soft converted, as shown in this specification, except for continuously reinforced pavement.

For English reinforcement or metric size bars which have been soft converted, substitution in continuously reinforced pavement, the following table shall be used in conjunction with Standard 421001, Bar Reinforcement for Continuously Reinforced PCC Pavement.

| Metric Project Information |  | May Use The English Bars For Metric Project at the Spacing and Dimensions Shown |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Metric Bar Size | Pavement Thickness |  | English Bar Sub- stitution | Metric Size Soft Converted (ASTM A 615 M - 96a) | A Approximate Spacing | B | C | D |
| #20 | 200 thru 220 |  | #6 | #19 | 18 spaces (19 bars)@ 190 | 100 | 80 | 560 |
| #20 | 230 thru 250 |  | #6 | #19 | 21 spaces (22 bars)@ 160 | 130 | 110 | 560 |
| #20 | 260 thru 280 |  | #6 | #19 | 23 spaces (24 bars)@ 145 | 140 | 125 | 560 |
| #20 | 290 thru 310 |  | #6 | #19 | 26 spaces (27 bars)@ 130 | 120 | 100 | 560 |
| #20 | 320 thru 340 |  | #6 | #19 | 29 spaces (30 bars)@ 115 | 140 | 125 | 560 |
| #25 | 230 thru 250 |  | #7 | #22 | 15 spaces (16 bars)@ 225 | 125 | 100 | 660 |
| #25 | 260 thru 280 |  | #7 | #22 | 17 spaces (18 bars)@ 200 | 110 | 90 | 660 |
| #25 | 290 thru 310 |  | #7 | #22 | 19 spaces (20 bars)@ 180 | 100 | 80 | 660 |
| #25 | 320 thru 340 |  | #7 | #22 | 21 spaces (22 bars)@ 160 | 130 | 110 | 660 |
| #25 | 350 thru 370 |  | #7 | #22 | 23 spaces (24 bars)@ 145 | 140 | 125 | 660 |
| #25 | 380 thru 400 |  | #7 | #22 | 25 spaces (26 bars)@ 135 | 125 | 100 | 660 |
| #25 | 410 thru 430 |  | #7 | #22 | 27 spaces (28 bars)@ 125 | 125 | 100 | 660 |

No additional payment will be made for any additional weight of steel furnished in substituting English size reinforcement bars or metric size reinforcement which have been soft converted for metric bars shown on the plans.

30770