All District Engineers, Carl Kowalski & Thomas R. Walker

Gary Gould

SPECIAL PROVISION FOR ENGLISH SUBSTITUTION OF METRIC BOLTS

May 1, 1996

This special provision was developed due to the difficulty in obtaining metric sized bolts.  The Bureau of Bridges and Structures developed this special provision to allow the Contractor to substitute English size bolts for metric size bolts on metric projects.

This special provision should be included in all metric projects containing metric bolts.

The Project Development and Implementation Section will include the subject special provision in all applicable projects scheduled for the July 12, 1996, letting.  Your office should include this special provision in all applicable projects for subsequent lettings.

This special provision will be transferred through the LAN system, to the district offices on May 28, 1996.

CK:rc\/tmp/in/input.doc

English Substitution of Metric Bolts (BDE)

Effective:  July 1, 1996

This special provision consists of giving the Contractor the option of replacing metric size bolts with English size bolts.

For ASTM A 325M and AASHTO M 164M, the following substitutions will be allowed:

Metric Bolt					English Substitution

Diameter (mm)				Diameter (inches)

| M16 | 5/8 |
| --- | --- |
| M22 | 7/8 |
| M27 | 1-1/8 |
| M30 | 1-1/4 |

A 3/4 inch diameter bolt may be substituted for a M20 bolt only on connections for straight multi-girder systems, detailed with over-sized holes.

For ASTM A 307, the following substitutions will be allowed:

Metric Bolt					English Substitution

Diameter (mm)				Diameter (inches)

| M24 | 1 |
| --- | --- |
| M30 | 1-1/4 |
| M36 | 1-1/2 |
| M48 | 2 |
| M64 | 2-1/2 |

41427