All Regional Engineers

Charles J. Ingersoll

Special Provision for Building Removal - Case IV (No Asbestos)

January 8, 2010

This special provision has been revised to make a minor clarification.  It should be included on contracts involving building removal with no asbestos abatement whatsoever.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the April 23, 2010 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory
January 8, 2010.

5053im

# Building Removal - Case IV (No Asbestos) (BDE)

Effective:  September 1, 1990

Revised:  April 1, 2010

BUILDING REMOVAL:  This work shall consist of the removal and disposal of       building(s), together with all foundations, retaining walls, and piers, down to a plane 1 ft (300 mm) below the ultimate or existing grade in the area and also all incidental and collateral work necessary to complete the removal of the building(s) in a manner approved by the Engineer.  Any holes, such as basements, shall be filled with a suitable granular material.  The building(s) are identified as follows:

Parcel

Bldg. No.		  No.   			Location			Description

Discontinuance of Utilities:  The Contractor shall arrange for the discontinuance of all utility services and the removal of the metering devices that serve the building(s) according to the respective requirements and regulations of the City, County, or utility companies involved.  The Contractor shall disconnect and seal, in an approved manner, all service outlets that serve any building(s) he/she is to remove.

Signs:  Immediately upon execution of the contract and prior to the wrecking of any structures, the Contractor shall be required to paint or stencil, in contrasting colors of an oil base paint, on all four sides of each residence and two opposite sides of other structures, the following sign:

PROPERTY ACQUIRED FOR

HIGHWAY  CONSTRUCTION

TO BE DEMOLISHED BY THE

VANDALS WILL BE PROSECUTED

The signs shall be positioned in a prominent location on the structure so that they can be easily seen and read and at a sufficient height to prevent defacing.  The Contractor shall not paint signs nor start demolition of any building(s) prior to the time that the State becomes the owner of the respective building(s).

Basis of Payment:  This work will be paid for at the contract lump sum unit price for BUILDING REMOVAL, numbers as listed above, which price shall be payment in full for complete removal of the buildings and structures, including any necessary backfilling material as specified herein.  The lump sum unit price(s) for this work shall represent the cost of demolition.  Any salvage value shall be reflected in the contract unit price for this item.

Notifications:  The "Demolition/Renovation Notice" form, which can be obtained from the IEPA office, shall be completed and submitted to the address listed below at least ten days prior to commencement of any demolition activity.

Asbestos Demolition/Renovation Coordinator

Illinois Environmental Protection Agency

Division of Air Pollution Control

P. O. Box 19276

Springfield, Illinois 62794-9276

(217)785-1743

Notices shall be updated if there is a change in the starting date or the amount of asbestos changes by more than 20 percent.

Submittals:

A.	All submittals and notices shall be made to the Engineer except where otherwise specified herein.

B.	Prior to starting work, the Contractor shall submit proof of written notification and compliance with the “Notifications” paragraph.

5053I