# GUARD POSTS

Eff. 03-05-1997 							Rev. 01-01-2014

This work shall consist of furnishing and setting guard posts according to Section 634 of the Standard Specifications except as follows:

- The cross section of the posts shall be nominal 4 inches by 4 inches (100 mm by 100 mm.)

- The tops of the posts shall not be rounded, but shall be sloped at 30 degrees to the horizontal.

- The length of the posts shall be 5 feet (1.5 m).

- The embedment of the posts shall be 33 inches (0.84 m.)

- For closure of median ditch checks, the nominal spacing of the posts shall be 5 feet (1.5 m.)

634