All Regional Engineers

Scott E. Stitt

Special Provision for Impact Attenuators, Temporary

September 30, 2011

This special provision was developed by the Bureau of Design and Environment.  It has been revised to incorporate MASH testing criteria.

This special provision should be included on all contracts using impact attenuators in a temporary application.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the January 20, 2012 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
September 30, 2011.

80110m

# impact attenuatorS, temporary (BDE)

Effective:  November 1, 2003

Revised:  January 1, 2012

Description.  This work shall consist of furnishing, installing, maintaining, and removing temporary impact attenuators of the category and test level specified.

Materials.  Materials shall be according to the impact attenuator manufacturer’s specifications and the following:

Item	Article/Section

(a)	Fine Aggregate (Note 1)	1003.01

(b)	Steel Posts, Structural Shapes, and Plates	1006.04

(c)	Rail Elements, End Section Plates, and Splice Plates	1006.25

(d)	Bolts, Nuts, Washers and Hardware	1006.25

(e)	Hollow Structural Tubing	1006.27(b)

(f)	Wood Posts and Wood Blockouts	1007.01, 1007.02, 1007.06

(g)	Preservative Treatment	1007.12

(h)	Packaged Rapid Hardening Mortar 	1018.01

Note 1.  Fine aggregate shall be FA 1 or FA 2, Class A quality.  The sand shall be unbagged and shall have a maximum moisture content of five percent.

CONSTRUCTION REQUIREMENTS

General.  Impact Attenuators shall meet the testing criteria contained in either the National Cooperative Highway Research Program (NCHRP) Report 350 or MASH and shall be on the Department’s approved list.

Installation.  Impact attenuators shall be installed according to the manufacturer’s specifications and include all necessary transitions between the impact attenuator and the item to which it is attached.  Regrading of slopes or approaches for the installation shall be as shown on the plans.

Attenuator bases, when required by the manufacturer, shall be constructed on a prepared subgrade according to the manufacturer’s specifications.  The surface of the base shall be slightly sloped or crowned to facilitate drainage.

When water filled attenuators are used between November 1 and April 15, they shall contain anti-freeze according to the manufacturer’s recommendations.

Markings.  Sand module impact attenuators shall be striped with alternating reflectorized Type AA or Type AP fluorescent orange and reflectorized white horizontal, circumferential stripes.  There shall be at least two of each stripe on each module.

Other types of impact attenuators shall have a terminal marker applied to their nose and reflectors along their sides.

Maintenance.  All maintenance of the impact attenuators shall be the responsibility of the Contractor until removal is directed by the Engineer.

Relocate.  When relocation of temporary impact attenuators is specified, they shall be removed, relocated and reinstalled at the new location.  The reinstallation requirements shall be the same as those for a new installation.

Removal.  When the Engineer determines the temporary impact attenuators are no longer required, the installation shall be dismantled with all hardware becoming the property of the Contractor.

Surplus material shall be disposed of according to Article 202.03.  Anti-freeze, when present, shall be disposed of/recycled according to local ordinances.

When impact attenuators have been anchored to the pavement, the anchor holes shall be repaired with rapid set mortar; only enough water to permit placement and consolidation by rodding shall be used and the material shall be struck-off flush.

Method of Measurement.  This work will be measured for payment as each, where each is defined as one complete installation.

Basis of Payment.  This work will be paid for at the contract unit price per each for IMPACT ATTENUATORS, TEMPORARY (FULLY REDIRECTIVE, NARROW); IMPACT ATTENUATORS, TEMPORARY (FULLY REDIRECTIVE, WIDE); IMPACT ATTENUATORS, TEMPORARY (FULLY REDIRECTIVE, RESETTABLE); IMPACT ATTENUATORS, TEMPORARY (SEVERE USE, NARROW); IMPACT ATTENUATORS, TEMPORARY (SEVERE USE, WIDE); or IMPACT ATTENUATORS, TEMPORARY (NON-REDIRECTIVE) of the test level specified.

Relocation of the devices will be paid for at the contract unit price per each for IMPACT ATTENUATORS, RELOCATE (FULLY REDIRECTIVE); IMPACT ATTENUATORS, RELOCATE (SEVERE USE); or IMPACT ATTENUATORS, RELOCATE (NON-REDIRECTIVE); of the test level specified.

Regrading of slopes or approaches will be paid for according to Section 202 and/or Section 204 of the Standard Specifications.

80110