All Regional Engineers

Eric E. Harm

Special Provision for Prismatic Curb Reflectors

July 25, 2008

This special provision was developed by the Bureau of Operations to provide additional information for prismatic curb reflectors.

This special provision should be used in contracts requiring prismatic curb reflectors.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 7, 2008, and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory July 25, 2008.

80211m

# prismatic curb reflectors (BDE)

Effective:  November 1, 2008

Add the following paragraph to the end of Article 782.03 of the Standard Specifications:

“The installed height of the prismatic curb reflectors shall be a maximum of 3/4 in. (19 mm) above the mounting surface.  The unit shall have one reflective surface that is placed approximately perpendicular to the mounting surface.”

Add the following Article to Section 1097 of the Standard Specifications:

“1097.04	Prismatic Curb Reflectors.  The unit shall provide a reflective area between 1 1/2 sq in. (960 sq mm) and 2 sq in. (1290 sq mm).  The base of the marker shall be designed for adhesive mounting.

The unit shall support an 800 lb (360 kg) load.  This shall be determined by placing the unit on a flat plate and slowly applying the load by means of another plate evenly to the entire top flat surface of the unit.  Breakage or significant deformation of the unit shall constitute failure.

The coefficient of luminous intensity of each reflector shall be equal to or exceed the following minimum values regardless of reflector orientation.

| Divergence Angle Degrees | Entrance Angle Degrees | Intensity Candle Power per Foot Candle (candelas/lux) |  |
| --- | --- | --- | --- |
|  |  | Crystal | Amber |
| 0.2º | 0º | 14 (1.3) | 11 (1.0) |
| 0.2º | +5º * | 14 (1.3) | 11 (1.0) |
| 0.2º | +10º * | 9 (0.8) | 7 (0.7) |
| 0.2º | +20º * | 5 (0.5) | 7 (0.4) |

* Traffic side”

80211