All Regional Engineers

Eric E. Harm

Special Provision for Partial Exit Ramp Closure for Freeway/Expressway

September 26, 2008

This special provision was developed by the Bureau of Operations and the Bureau of Design and Environment to provide statewide requirements for partial closure of exit ramps on freeway/expressways.  It should be inserted into contracts using Highway Design Standard 701456.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the January 16, 2009 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory September 26, 2008.

80216m

# Partial exit ramp closure for freeway/expressway (BDE)

Effective:  January 1, 2009

Description.  This work shall consist of furnishing and installing traffic control for the partial closure of exit ramps on a freeway/expressway.  Work shall be according to Section 701 except as modified herein.

Add the following after the fourth paragraph of Article 701.07 of the Standard Specifications:

“Drop-offs at the edge of pavement greater than 1 1/2 in. (40 mm) caused by the Contractor’s operations will be allowed only on one side of the ramp at a time.”

Delete the third paragraph of Article 701.17(e)(1) of the Standard Specifications.

Delete the third paragraph of Article 701.18(e)(3) of the Standard Specifications.

Revise the first sentence of Article 701.19(c) of the Standard Specifications to read:

“Traffic control and protection required under Standards 701201, 701206, 701306, 701326, 701336, 701406, 701421, 701456, 701501, 701502, 701601, 701602, 701606, 701701 and 701801 will be measured for payment on a lump sum basis.”

Add the following to the first paragraph of Article 701.20(b) of the Standard Specifications:

“TRAFFIC CONTROL AND PROTECTION STANDARD 701456;”

80216