All Regional Engineers

Scott E. Stitt

Special Provision for Sidewalk, Corner or Crosswalk Closure

September 30, 2011

This special provision was developed by the Bureau of Design and Environment to establish requirements for detectable pedestrian channelizing barricades.

It should be included in all projects using Highway Standard 701801.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the January 20, 2012 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
September 30, 2011.

80285m

# sidewalk, corner or crosswalk closure (bde)

Effective:  January 1, 2012

Add the following to Article 701.03 of the Standard Specifications:

“	(p)	Detectable Pedestrian Channelizing Barricades 	1106.02(k)”

Add the following to Article 701.15 of the Standard Specifications:

“(n)	Detectable Pedestrian Channelizing Barricade.  Detectable pedestrian channelizing barricades are cane detectable and visible to persons having low vision.  These barricades are used to channelize pedestrian traffic.”

Add the following to Article 1106.02 of the Standard Specifications:

“(m)	Detectable Pedestrian Channelizing Barricades.  The top and bottom panels shall have alternating white and orange stripes sloping at 45 degrees on the side exposed to pedestrian traffic.  Barricade stripes shall be 6 in. (150 mm) in width.  The predominant color for other barricade components shall be white, orange, or silver.

The top and bottom rails shall be continuous to allow for detection for hand trailing and cane trailing, respectively.

The faces of the barricade rails shall be vertical.”

80285