All Regional Engineers

John D. Baranzelli

Special Provision for Seeding

July 27, 2012

This special provision was developed by the Bureau of Design and Environment to address the omission of the note requiring pure live seed (PLS) for native seed varieties in Seeding, Class 3.

This special provision should be inserted into contracts with Seeding, Class 3.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 9, 2012 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
July 27, 2012.

80307m

# Seeding (bde)

Effective:  November 1, 2012

Revise the following seeding mixture shown in Table 1 of Article 250.07 of the Standard Specifications to read.

| “TABLE 1 - SEEDING MIXTURES |  |  |  |
| --- | --- | --- | --- |
| Class - Type |  | Seeds | lb/acre (kg/hectare) |
| 3 | Northern Illinois Slope Mixture 7/ | Elymus Canadensis (Canada Wild Rye) 5/ | 5 (5) |
|  |  | Perennial Ryegrass | 20 (20) |
|  |  | Alsike Clover 2/ | 5 (5) |
|  |  | Desmanthus Illinoensis (Illinois Bundleflower) 2/, 5/ | 2 (2) |
|  |  | Andropogon Scoparius |  |
|  |  | (Little Bluestem) 5/ | 12 (12) |
|  |  | Bouteloua Curtipendula |  |
|  |  | (Side-Oats Grama) 5/ | 10 (10) |
|  |  | Fult Salt Grass 1/ | 30 (35) |
|  |  | Oats, Spring | 50 (55) |
|  |  | Slender Wheat Grass 5/ | 15 (15) |
|  |  | Buffalo Grass (Cody or Bowie) 4/, 5/, 9/ | 5 (5)” |

80307