All Regional Engineers

John D. Baranzelli

Special Provision for Concrete End Sections for Pipe Culverts

September 28, 2012

This special provision was developed by the Bureau of Design and Environment to establish construction requirements, a method of measurement and a basis of payment for the new Highway Standards for concrete end sections for pipe culverts.

This special provision should be inserted into contracts utilizing any of the following Highway Standards:  542001, 542006, 542011, or 542016.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the January 18, 2013 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
September 28, 2012.

80311m

# concrete end sections for pipe culverts (bde)

Effective:  January 1, 2013

Description.  This work shall consist of constructing cast-in-place concrete and precast concrete end sections for pipe culverts.  These end sections are shown on the plans as Highway Standard 542001, 542006, 542011, or 542016.  This work shall be according to Section 542 of the Standard Specifications except as modified herein.

Materials.  Materials shall be according to the following Articles of Division 1000 – Materials of the Standard Specifications.

Item	Article/Section

(a)	Portland Cement Concrete (Note 1) 	1020

(b)	Precast Concrete End Sections (Note 2)

(c)	Coarse Aggregate (Note 3) 	1004.05

(d)	Structural Steel (Note 4) 	1006.04

(e)	Anchor Bolts and Rods (Note 5) 	1006.09

(f)	Reinforcement Bars 	1006.10(a)

(g)	Nonshrink Grout 	1024.02

(h)	Chemical Adhesive Resin System 	1027

(i)	Mastic Joint Sealer for Pipe 	1055

(j)	Hand Hole Plugs 	1042.16

Note 1.  Cast-in-place concrete end sections shall be Class SI, except the 14 day mix design shall have a compressive strength of 5000 psi (34,500 kPa) or a flexural strength of (800 psi) 5500 kPa and a minimum cement factor of 6.65 cwt/cu yd (395 kg/cu m).

Note 2.  Precast concrete end sections shall be according to Articles 1042.02 and 1042.03(b)(c)(d)(e) of the Standard Specifications.  The concrete shall be Class PC according to Section 1020, and shall have a minimum compressive strength of 5000 psi (34,000 kPa) at 28 days.

Joints between precast sections shall be produced with reinforced tongue and groove ends according to the requirements of ASTM C 1577.

Note 3.  The granular bedding placed below a precast concrete end section shall be gradation CA 6, CA 9, CA 10, CA 12, CA 17, CA 18, or CA 19.

Note 4.  All components of the culvert tie detail shall be galvanized according to the requirements of AASHTO M 111 or M 232 as applicable.

Note 5.  The anchor rods for the culvert ties shall be according to the requirements of ASTM F 1554, Grade 105 (Grade 725).

CONSTRUCTION REQUIREMENTS

The concrete end sections may be precast or cast-in-place construction.  Toe walls shall be either precast or cast-in-place, and shall be in proper position and backfilled according to the applicable paragraphs of Article 502.10 of the Standard Specifications prior to the installation of the concrete end sections.  If soil conditions permit, cast-in-place toe walls may be poured directly against the soil.  When poured directly against the soil, the clear cover of the sides and bottom of the toe wall shall be increased to 3 in. (75 mm) by increasing the thickness of the toe wall.

(a)	Cast-In-Place Concrete End Sections.  Cast-in-place concrete end sections shall be constructed according to the requirements of Section 503 of the Standard Specifications and as shown on the plans.

(b)	Precast Concrete End Sections.  When the concrete end sections will be precast, shop drawings detailing the slab thickness and reinforcement layout shall be submitted to the Engineer for review and approval.

The excavation and backfilling for precast concrete end sections shall be according to the requirements of Section 502 of the Standard Specifications, except a layer of granular bedding at least 6 in. (150 mm) in thickness shall be placed below the elevation of the bottom of the end section.  The granular bedding shall extend a minimum of 2 ft (600 mm) beyond each side of the end section.

Anchor rods connecting precast sections shall be brought to a snug tight condition followed by an additional 2/3 turn on one of the nuts.  Match marks shall be provided on the bolt and nut to verify relative rotation between the bolt and the nut.

Method of Measurement.  This work will be measured for payment as each, with each end of each culvert being one each.

Basis of Payment.  This work will be paid for at the contract unit price per each for CONCRETE END SECTION, STANDARD 542001; CONCRETE END SECTION, STANDARD 542006; CONCRETE END SECTION, 542011; or CONCRETE END SECTION, 542016, of the pipe diameter and slope specified.

80311