All Regional Engineers

John D. Baranzelli

Special Provision for Removal and Disposal of Surplus Materials

January 11, 2013

This special provision was developed by the Bureau of Design and Environment, as a result of Clean Construction or Demolition Debris (CCDD) rules issued by the EPA.  This special provision updates Article 202.03 with the EPA’s pH requirements for soil.

It should be included in all projects involving excavation and grading.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the April 26, 2013 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
January 11, 2013.

80319m

# REMOVAL AND DISPOSAL OF SURPLUS MATERIALS (BDE)

Effective:  November 2, 2012

Revise the first four paragraphs of Article 202.03 of the Standard Specifications to read:

“	202.03	Removal and Disposal of Surplus, Unstable, Unsuitable, and Organic Materials.  Suitable excavated materials shall not be wasted without permission of the Engineer.  The Contractor shall dispose of all surplus, unstable, unsuitable, and organic materials, in such a manner that public or private property will not be damaged or endangered.

Suitable earth, stones and boulders naturally occurring within the right-of-way may be placed in fills or embankments in lifts and compacted according to Section 205.  Broken concrete without protruding metal bars, bricks, rock, stone, reclaimed asphalt pavement with no expansive aggregate, or uncontaminated dirt and sand generated from construction or demolition activities may be used in embankment or in fill.  If used in fills or embankments, these materials shall be placed and compacted to the satisfaction of the Engineer; shall be buried under a minimum of 2 ft (600 mm) of earth cover (except when the materials include only uncontaminated dirt); and shall not create an unsightly appearance or detract from the natural topographic features of an area.  Broken concrete without protruding metal bars, bricks, rock, or stone may be used as riprap as approved by the Engineer.  If the materials are used for fill in locations within the right-of-way but outside project construction limits, the Contractor must specify to the Engineer, in writing, how the landscape restoration of the fill areas will be accomplished.  Placement of fill in such areas shall not commence until the Contractor’s landscape restoration plan is approved by the Engineer.

Aside from the materials listed above, all other construction and demolition debris or waste shall be disposed of in a licensed landfill, recycled, reused, or otherwise disposed of as allowed by State or Federal laws and regulations.  When the Contractor chooses to dispose of uncontaminated soil at a clean construction and demolition debris (CCDD) facility or at an uncontaminated soil fill operation, it shall be the Contractor’s responsibility to have the pH of the material tested to ensure the value is between 6.25 and 9.0, inclusive.  A copy of the pH test results shall be provided to the Engineer.

A permit shall be obtained from IEPA and made available to the Engineer prior to open burning of organic materials (i.e., plant refuse resulting from pruning or removal of trees or shrubs) or other construction or demolition debris.  Organic materials originating within the right-of-way limits may be chipped or shredded and placed as mulch around landscape plantings within the right-of-way when approved by the Engineer.  Chipped or shredded material to be placed as mulch shall not exceed a depth of 6 in. (150 mm).”

80319