All Regional Engineers

Omer M. Osman, P.E.

Special Provision for LRFD Pipe Culvert Burial Tables

July 25, 2014

This special provision was developed by the Bureau of Bridges and Structures as a result of updating the pipe culvert burial tables to be compliant with the AASHTO LRFD Design Code along with updating the pipe materials available.

Highlights of the changes include:

- LRFD compliance for all tables and materials.

- Upgraded to 75 year design life.

- Added CPP flexible pipe material.

- Removed profile wall PVC and profile wall PE flexible pipe materials.

- Added D loads for special design concrete pipe to Tables IA and IIB.

- Added 5 x 1 corrugation to Table IB.

- Added 5 x 1 steel and 3 x 1 aluminum corrugations to Table IIA.

- Deleted Table IV.

This special provision has been revised to:

- Update terminology for various metal pipe materials.

- Require a higher degree of corrosion protection for corrugated steel pipes for diameters up to and including 42 inch.

- Introduce the option for helical seam fabrication.

- Correct other minor issues.

This special provision should be inserted into contracts involving pipe culvert installation.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 7, 2014 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory July 25, 2014.

80324m

# lrfd pipe culvert burial tables (bde)

Effective:  November 1, 2013

Revised:  November 1, 2014

Revise Article 542.02 of the Standard Specifications to read as follows:

“Item	Article/Section

(a)	Galvanized Corrugated Steel Pipe 	1006.01

(b)	Galvanized Corrugated Steel Pipe Arch 	1006.01

(c)	Bituminous Coated Corrugated Steel Pipe 	1006.01

(d)	Bituminous Coated Corrugated Steel Pipe Arch 	1006.01

(e)		Reserved

(f)	Aluminized Steel Type 2 Corrugated Pipe 	1006.01

(g)	Aluminized Steel Type 2 Corrugated Pipe Arch 	1006.01

(h)	Precoated Galvanized Corrugated Steel Pipe 	1006.01

(i)	Precoated Galvanized Corrugated Steel Pipe Arch 	1006.01

(j)	Corrugated Aluminum Alloy Pipe 	1006.03

(k)	Corrugated Aluminum Alloy Pipe Arch 	1006.03

(l)	Extra Strength Clay Pipe 	1040.02

(m)	Concrete Sewer, Storm Drain, and Culvert Pipe 	1042

(n)	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe 	1042

(o)	Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe	1042

(p)	Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe 	1042

(q)	Polyvinyl Chloride (PVC) Pipe 	1040.03

(r)	Corrugated Polyvinyl Chloride (PVC) Pipe with a Smooth Interior 	1040.03

(s)	Corrugated Polypropylene (CPP) pipe with smooth Interior 	1040.07

(t)	Corrugated Polyethylene (PE) Pipe with a Smooth Interior 	1040.04

(u)	Polyethylene (PE) Pipe with a Smooth Interior 	1040.04

(v)	Rubber Gaskets and Preformed Flexible Joint Sealants for Concrete Pipe 	1056

(w)	Mastic Joint Sealer for Pipe 	1055

(x)	External Sealing Band 	1057

(y)	Fine Aggregate (Note 1) 	1003.04

(z)	Coarse Aggregate (Note 2) 	1004.05

(aa)	Packaged Rapid Hardening Mortar or Concrete 	1018

(bb)	Nonshrink Grout 	1024.02

(cc)	Reinforcement Bars and Welded Wire Fabric 	1006.10

(dd)	Handling Hole Plugs 	1042.16

Note 1.	The fine aggregate shall be moist.

Note 2.	The coarse aggregate shall be wet.”

Revise the table for permitted materials in Article 542.03 of the Standard Specifications as follows:

| ### “Class | ### Materials |
| --- | --- |
| A | Rigid Pipes: |
|  | Extra Strength Clay Pipe |
|  | Concrete Sewer Storm Drain and Culvert Pipe, Class 3 |
|  | Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe |
| C | Rigid Pipes: |
|  | Extra Strength Clay Pipe |
|  | Concrete Sewer Storm Drain and Culvert Pipe, Class 3 |
|  | Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe |
|  | Flexible Pipes: |
|  | Aluminized Steel Type 2 Corrugated Pipe |
|  | Aluminized Steel Type 2 Corrugated Pipe Arch |
|  | Precoated Galvanized Corrugated Steel Pipe |
|  | Precoated Galvanized Corrugated Steel Pipe Arch |
|  | Corrugated Aluminum Alloy Pipe |
|  | Corrugated Aluminum Alloy Pipe Arch |
|  | Polyvinyl Chloride (PVC) Pipe |
|  | Corrugated Polyvinyl Chloride (PVC) Pipe with a Smooth Interior |
|  | Polyethylene (PE) Pipe with a Smooth Interior |
|  | Corrugated Polypropylene (CPP) Pipe with Smooth Interior |
| D | Rigid Pipes: |
|  | Extra Strength Clay Pipe |
|  | Concrete Sewer Storm Drain and Culvert Pipe, Class 3 |
|  | Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe |
|  | Flexible Pipes: |
|  | Galvanized Corrugated Steel Pipe |
|  | Galvanized Corrugated Steel Pipe Arch |
|  | Bituminous Coated Corrugated Steel Pipe |
|  | Bituminous Coated Corrugated Steel Pipe Arch |
|  | Aluminized Steel Type 2 Corrugated Pipe |
|  | Aluminized Steel Type 2 Corrugated Pipe Arch |
|  | Precoated Galvanized Corrugated Steel Pipe |
|  | Precoated Galvanized Corrugated Steel Pipe Arch |
|  | Corrugated Aluminum Alloy Pipe |
|  | Corrugated Aluminum Alloy Pipe Arch |
|  | Polyvinyl Chloride (PVC) Pipe |
|  | Corrugated Polyvinyl Chloride (PVC) Pipe with a Smooth Interior |
|  | Corrugated Polyethylene (PE) Pipe with a Smooth Interior |
|  | Polyethylene (PE) Pipe with a Smooth Interior” |
|  | Corrugated Polypropylene (CPP) Pipe with Smooth Interior |

Revise Articles 542.03(b) and (c) of the Standard Specifications to read:

“	(b)	Extra strength clay pipe will only be permitted for pipe culverts Type 1, for 10 in., 12 in., 42 in. and 48 in. (250 mm, 300 mm, 1050 mm and 1200 mm), Types 2, up to and including 48 in. (1200 mm), Type 3, up to and including 18 in. (450 mm), Type 4 up to and including 10 in. (250 mm), for all pipe classes.

(c)	Concrete sewer, storm drain, and culvert pipe Class 3 will only be permitted for pipe culverts Type 1, up to and including 10 in (250 mm), Type 2, up to and including 30 in. (750 mm), Type 3, up to and including 15 in. (375 mm); Type 4, up to and including 10 in. (250 mm), for all pipe classes.”

Replace the pipe tables in Article 542.03 of the Standard Specifications with the following:

| “Table IA: Classes of Reinforced Concrete Pipe for the Respective Diameters of Pipe and Fill Heights over the Top of the Pipe |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 1 | Type 2 | Type 3 | Type 4 | Type 5 | Type 6 | Type 7 |
|  | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: |
|  | 3' and less 1' min cover | Greater than 3' not exceeding 10' | Greater than 10' not exceeding 15' | Greater than 15' not exceeding 20' | Greater than 20' not exceeding 25' | Greater than 25' not exceeding 30' | Greater than 30' not exceeding 35' |
| 12 | IV | II | III | IV | IV | V | V |
| 15 | IV | II | III | IV | IV | V | V |
| 18 | IV | II | III | IV | IV | V | V |
| 21 | III | II | III | IV | IV | V | V |
| 24 | III | II | III | IV | IV | V | V |
| 30 | IV | II | III | IV | IV | V | V |
| 36 | III | II | III | IV | IV | V | V |
| 42 | II | II | III | IV | IV | V | V |
| 48 | II | II | III | IV | IV | V | V |
| 54 | II | II | III | IV | IV | V | V |
| 60 | II | II | III | IV | IV | V | V |
| 66 | II | II | III | IV | IV | V | V |
| 72 | II | II | III | IV | V | V | V |
| 78 | II | II | III | IV | 2020 | 2370 | 2730 |
| 84 | II | II | III | IV | 2020 | 2380 | 2740 |
| 90 | II | II | III | 1680 | 2030 | 2390 | 2750 |
| 96 | II | III | III | 1690 | 2040 | 2400 | 2750 |
| 102 | II | III | III | 1700 | 2050 | 2410 | 2760 |
| 108 | II | III | 1360 | 1710 | 2060 | 2410 | 2770 |

Notes:

A number indicates the D-Load for the diameter and depth of fill and that a special design is required.

Design assumptions; Water filled pipe, Type 2 bedding and Class C Walls

| Table IA: Classes of Reinforced Concrete Pipe for the Respective Diameters of Pipe and Fill Heights over the Top of the Pipe (Metric) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter mm | Type 1 | Type 2 | Type 3 | Type 4 | Type 5 | Type 6 | Type 7 |
|  | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: | Fill Height: |
|  | 1 m and less 0.3 m min cover | Greater than 1 m not exceeding 3 m | Greater than 3 m not exceeding 4.5 m | Greater than 4.5 m not exceeding 6 m | Greater than 6 m not exceeding 7.5 m | Greater than 7.5 m not exceeding 9 m | Greater than 9 m not exceeding 10.5 m |
| 300 | IV | II | III | IV | IV | V | V |
| 375 | IV | II | III | IV | IV | V | V |
| 450 | IV | II | III | IV | IV | V | V |
| 525 | III | II | III | IV | IV | V | V |
| 600 | III | II | III | IV | IV | V | V |
| 750 | IV | II | III | IV | IV | V | V |
| 900 | III | II | III | IV | IV | V | V |
| 1050 | II | II | III | IV | IV | V | V |
| 1200 | II | II | III | IV | IV | V | V |
| 1350 | II | II | III | IV | IV | V | V |
| 1500 | II | II | III | IV | IV | V | V |
| 1650 | II | II | III | IV | IV | V | V |
| 1800 | II | II | III | IV | V | V | V |
| 1950 | II | II | III | IV | 100 | 110 | 130 |
| 2100 | II | II | III | IV | 100 | 110 | 130 |
| 2250 | II | II | III | 80 | 100 | 110 | 130 |
| 2400 | II | III | III | 80 | 100 | 110 | 130 |
| 2550 | II | III | III | 80 | 100 | 120 | 130 |
| 2700 | II | III | 70 | 80 | 100 | 120 | 130 |

Notes:

A number indicates the D-Load for the diameter and depth of fill and that a special design is required.

Design assumptions; Water filled pipe, Type 2 bedding and Class C Walls

| TABLE IB: THICKNESS OF CORRUGATED STEEL PIPE FOR THE RESPECTIVE DIAMETER OF PIPE AND FILL HEIGHTS OVER THE TOP OF THE PIPE FOR 2 2/3"x1/2", 3"x1" AND 5"x1" CORRUGATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in.* | Type 1 |  |  | Type 2 |  |  | Type 3 |  |  | Type 4 |  |  | Type 5 |  |  | Type 6 |  |  | Type 7 |  |  |
|  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  |
|  | 3' and less 1' min. cover |  |  | Greater than 3' not exceeding 10' |  |  | Greater than 10' not exceeding 15' |  |  | Greater than 15' not exceeding 20' |  |  | Greater than 20' not exceeding 25' |  |  | Greater than 25' not exceeding 30' |  |  | Greater than 30' not exceeding 35' |  |  |
|  | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" | 2 2/3" x 1/2" | 3"x1" | 5"x1" |
| 12 | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  |
| 15 | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | (0.079) |  |  |
| 18 | (0.079) |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  |
| 21 | (0.079) |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  | (0.079) |  |  |
| 24 | (0.079) |  |  | 0.064 |  |  | 0.064 |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  | (0.109) |  |  |
| 30 | (0.109E) |  |  | 0.064 |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  | (0.109) |  |  | 0.109 |  |  |
| 36 | (0.109E) |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  | (0.109) |  |  | 0.109 |  |  | (0.138E) |  |  |
| 42 | 0.079 |  |  | 0.064 |  |  | (0.079) |  |  | (0.079) |  |  | (0.109) |  |  | (0.109E) |  |  | (0.109E) |  |  |
| 48 | 0.109 | (0.109) | 0.109 | (0.109) | 0.079 | 0.079 | (0.109) | 0.079 | (0.109) | 0.109 | (0.109) | 0.109 | (0.138) | (0.109) | 0.109 | (0.138E) | 0.109 | 0.109 | (0.138E) | 0.109 | (0.138) |
| 54 | 0.109 | (0.109) | 0.109 | (0.109) | 0.079 | 0.079 | 0.109 | (0.109) | 0.109 | 0.109 | (0.109) | 0.109 | (0.138) | 0.109 | 0.109 | (0.138E) | 0.109 | (0.138) | (0.138E) | 0.138 | 0.138 |
| 60 | 0.109 | 0.109 | 0.109 | 0.109 | 0.079 | (0.109) | 0.109 | (0.109) | 0.109 | 0.109 | (0.109) | 0.109 | (0.138) | 0.109 | 0.109 | (0.138E) | (0.138) | (0.138) | 0.138E | (0.138E) | (0.138E) |
| 66 | (0.138) | 0.109 | 0.109 | 0.109 | 0.079 | (0.109) | 0.109 | (0.109) | 0.109 | 0.109 | 0.109 | 0.109 | (0.138) | 0.109 | (0.138) | (0.138E) | 0.138 | 0.138 | 0.138E | (0.138E) | 0.138E |
| 72 | 0.138 | 0.109 | (0.138) | 0.138 | (0.109) | (0.109) | 0.138 | (0.109) | 0.109 | 0.138 | 0.109 | 0.109 | 0.138 | (0.138) | (0.138) | (0.168E) | (0.138E) | 0.138E | (0.168E) | (0.138E) | 0.138E |
| 78 | 0.168 | 0.109 | (0.138) | 0.168 | (0.109) | 0.109 | 0.168 | 0.109 | 0.109 | 0.168 | 0.109 | (0.138) | 0.168 | (0.138) | (0.138) | H0.168E | (0.138E) | 0.138E | H0.168E | 0.138E | (0.168E) |
| 84 | 0.168 | (0.138) | (0.138) | 0.168 | (0.109) | 0.109 | 0.168 | 0.109 | 0.109 | 0.168 | 0.109 | (0.138) | 0.168 | (0.138) | 0.138 | H0.168E | (0.138E) | 0.138E | H0.168E | (0.168E) | (0.168E) |
| 90 |  | (0.138) | (0.138) |  | (0.109) | 0.109 |  | 0.109 | 0.109 |  | (0.138) | (0.138) |  | (0.138) | 0.138 |  | 0.138E | (0.168E) |  | (0.168E) | (0.168E) |
| 96 |  | (0.138) | (0.138) |  | (0.109) | 0.109 |  | 0.109 | 0.109 |  | (0.138) | (0.138) |  | (0.138) | 0.138 |  | (0.168E) | (0.168E) |  | (0.168E) | (0.168E) |
| 102 |  | 0.109Z | 0.109Z |  | (0.109) | 0.109 |  | 0.109 | (0.138) |  | (0.138) | (0.138) |  | (0.138) | 0.138 |  | (0.168E) | (0.168E) |  | H0.138E | H0.168E |
| 108 |  | 0.109Z | (0.138Z) |  | 0.109 | 0.109 |  | 0.109 | (0.138) |  | (0.138) | 0.138 |  | 0.138 | (0.168) |  | (0.168E) | (0.168E) |  | H0.138E | H0.168E |
| 114 |  | 0.109Z | (0.138Z) |  | 0.109 | 0.109 |  | 0.109 | (0.138) |  | (0.138) | 0.138 |  | (0.168) | (0.168) |  | (0.168E) | 0.168E |  | H0.138E | H0.168E |
| 120 |  | 0.109Z | (0.138Z) |  | 0.109 | 0.109 |  | (0.138) | (0.138) |  | (0.138) | 0.138 |  | (0.168) | (0.168) |  | H0.138E | H0.168E |  | H0.168E | H0.168E |
| 126 |  | 0.138Z | 0.138Z |  | 0.138 | 0.138 |  | 0.138 | 0.138 |  | 0.138 | (0.168) |  | (0.168) | (0.168) |  | H0.138E | H0.168E |  | H0.168E | H0.168E |
| 132 |  | 0.138Z | 0.138Z |  | 0.138 | 0.138 |  | 0.138 | 0.138 |  | (0.168) | (0.168) |  | 0.168 | 0.168 |  | H0.138E | H0.168E |  | H0.168E | H0.168E |
| 138 |  | 0.138Z | 0.138Z |  | 0.138 | 0.138 |  | 0.138 | 0.138 |  | (0.168) | (0.168) |  | (0.168E) | H0.168E |  | H0.168E | H0.168E |  | H0.168E |  |
| 144 |  | 0.168Z | 0.168Z |  | 0.168 | 0.168 |  | 0.168 | 0.168 |  | 0.168 | 0.168 |  | H0.168E | H0.168E |  | H0.168E | H0.168E |  | H0.168E |  |

Notes:

*	Aluminized Type 2 Steel or Precoated Galvanized Steel shall be required for diameters up to 42” according to Article 1006.01, 1 1/2" x 1/4" corrugations shall be used for diameters less than 12”.

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

A thickness preceded by “H” indicates only helical seam fabrication is allowed.

E	Elongation according to Article 542.04(e)

Z	1’-6” Minimum fill

| TABLE IB: THICKNESS OF CORRUGATED STEEL PIPE FOR THE RESPECTIVE DIAMETER OF PIPE AND FILL HEIGHTS OVER THE TOP OF THE PIPE FOR 68 mm x 13 mm, 75 mm x 25 mm AND 125 mm x 25 mm CORRUGATIONS (Metric) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter mm * | Type 1 |  |  | Type 2 |  |  | Type 3 |  |  | Type 4 |  |  | Type 5 |  |  | Type 6 |  |  | Type 7 |  |  |
|  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  | Fill Height: |  |  |
|  | 1 m and less 0.3 m min. cover |  |  | Greater than 1 m not exceeding 3 m |  |  | Greater than 3 m not exceeding 4.5 m |  |  | Greater than 4.5 m not exceeding 6 m |  |  | Greater than 6 m not exceeding 7.5 m |  |  | Greater than 7.5 m not exceeding 9 m |  |  | Greater than 9 m not exceeding 10.5 m |  |  |
|  | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm |
| 300 | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  |
| 375 | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | (2.01) |  |  |
| 450 | (2.01) |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  |
| 525 | (2.01) |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  | (2.01) |  |  |
| 600 | (2.01) |  |  | 1.63 |  |  | 1.63 |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  | (2.77) |  |  |
| 750 | (2.77E) |  |  | 1.63 |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  | (2.77) |  |  | 2.77 |  |  |
| 900 | (2.77E) |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  | (2.77) |  |  | 2.77 |  |  | (3.51E) |  |  |
| 1050 | 2.01 |  |  | 1.63 |  |  | (2.01) |  |  | (2.01) |  |  | (2.77) |  |  | (2.77E) |  |  | (2.77E) |  |  |
| 1200 | 2.77 | (2.77) | 2.77 | (2.77) | 2.01 | 2.01 | (2.77) | 2.01 | (2.77) | 2.77 | (2.77) | 2.77 | (3.51) | (2.77) | 2.77 | (3.51E) | 2.77 | 2.77 | (3.51E) | 2.77 | (3.51) |
| 1350 | 2.77 | (2.77) | 2.77 | (2.77) | 2.01 | 2.01 | 2.77 | (2.77) | 2.77 | 2.77 | (2.77) | 2.77 | (3.51) | 2.77 | 2.77 | (3.51E) | 2.77 | (3.51) | (3.51E) | 3.51 | 3.51 |
| 1500 | 2.77 | 2.77 | 2.77 | 2.77 | 2.01 | (2.77) | 2.77 | (2.77) | 2.77 | 2.77 | (2.77) | 2.77 | (3.51) | 2.77 | 2.77 | (3.51E) | (3.51) | (3.51) | 3.51E | (3.51E) | (3.51E) |
| 1650 | (3.51) | 2.77 | 2.77 | 2.77 | 2.01 | (2.77) | 2.77 | (2.77) | 2.77 | 2.77 | 2.77 | 2.77 | (3.51) | 2.77 | (3.51) | (3.51E) | 3.51 | 3.51 | 3.51E | (3.51E) | 3.51E |
| 1800 | 3.51 | 2.77 | (3.51) | 3.51 | (2.77) | (2.77) | 3.51 | (2.77) | 2.77 | 3.51 | 2.77 | 2.77 | 3.51 | (3.51) | (3.51) | (4.27E) | (3.51E) | 3.51E | (4.27E) | (3.51E) | 3.51E |
| 1950 | 4.27 | 2.77 | (3.51) | 4.27 | (2.77) | 2.77 | 4.27 | 2.77 | 2.77 | 4.27 | 2.77 | (3.51) | 4.27 | (3.51) | (3.51) | H 4.27E | (3.51E) | 3.51E | H 4.27E | 3.51E | (4.27E) |
| 2100 | 4.27 | (3.51) | (3.51) | 4.27 | (2.77) | 2.77 | 4.27 | 2.77 | 2.77 | 4.27 | 2.77 | (3.51) | 4.27 | (3.51) | 3.51 | H 4.27E | (3.51E) | 3.51E | H 4.27E | (4.27E) | (4.27E) |
| 2250 |  | (3.51) | (3.51) |  | (2.77) | 2.77 |  | 2.77 | 2.77 |  | (3.51) | (3.51) |  | (3.51) | 3.51 |  | 3.51E | (4.27E) |  | (4.27E) | (4.27E) |
| 2400 |  | (3.51) | (3.51) |  | (2.77) | 2.77 |  | 2.77 | 2.77 |  | (3.51) | (3.51) |  | (3.51) | 3.51 |  | (4.27E) | (4.27E) |  | (4.27E) | (4.27E) |
| 2550 |  | 2.77Z | 2.77Z |  | (2.77) | 2.77 |  | 2.77 | (3.51) |  | (3.51) | (3.51) |  | (3.51) | 3.51 |  | (4.27E) | (4.27E) |  | H 3.51E | H 4.27E |
| 2700 |  | 2.77Z | (3.51Z) |  | 2.77 | 2.77 |  | 2.77 | (3.51) |  | (3.51) | 3.51 |  | 3.51 | (4.27) |  | (4.27E) | (4.27E) |  | H 3.51E | H 4.27E |
| 2850 |  | 2.77Z | (3.51Z) |  | 2.77 | 2.77 |  | 2.77 | (3.51) |  | (3.51) | 3.51 |  | (4.27) | (4.27) |  | (4.27E) | 4.27E |  | H 3.51E | H 4.27E |
| 3000 |  | 2.77Z | (3.51Z) |  | 2.77 | 2.77 |  | (3.51) | (3.51) |  | (3.51) | 3.51 |  | (4.27) | (4.27) |  | H 3.51E | H 4.27E |  | H 4.27E | H 4.27E |
| 3150 |  | 3.51Z | 3.51Z |  | 3.51 | 3.51 |  | 3.51 | 3.51 |  | 3.51 | (4.27) |  | (4.27) | (4.27) |  | H 3.51E | H 4.27E |  | H 4.27E | H 4.27E |
| 3300 |  | 3.51Z | 3.51Z |  | 3.51 | 3.51 |  | 3.51 | 3.51 |  | (4.27) | (4.27) |  | 4.27 | 4.27 |  | H 3.51E | H 4.27E |  | H 4.27E | H 4.27E |
| 3450 |  | 3.51Z | 3.51Z |  | 3.51 | 3.51 |  | 3.51 | 3.51 |  | (4.27) | (4.27) |  | (4.27E) | H 4.27E |  | H 4.27E | H 4.27E |  | H 4.27E |  |
| 3600 |  | 4.27Z | 4.27Z |  | 4.27 | 4.27 |  | 4.27 | 4.27 |  | 4.27 | 4.27 |  | H 4.27E | H 4.27E |  | H 4.27E | H 4.27E |  | H 4.27E |  |

Notes:

*	Aluminized Type 2 Steel or Precoated Galvanized Steel shall be required for diameters up to 1050 mm according to Article 1006.01, 38 mm x 6.5 mm corrugations shall be used for diameters less than 300 mm.

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

A thickness preceded by an “H” indicates only helical seam fabrication is allowed.

E	Elongation according to Article 542.04(e)

Z	450 mm Minimum Fill

| TABLE IC: THICKNESS OF CORRUGATED ALUMINUM ALLOY PIPE FOR THE RESPECTIVE DIAMETER OF PIPE AND FILL HEIGHTS OVER THE TOP OF THE PIPE FOR 2 2/3"x1/2" AND 3"x1" CORRUGATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 1 |  | Type 2 |  | Type 3 |  | Type 4 |  | Type 5 |  | Type 6 |  | Type 7 |  |
|  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  |
|  | 3' and less 1' min. cover |  | Greater than 3' not exceeding 10' |  | Greater than 10' not exceeding 15' |  | Greater than 15' not exceeding 20' |  | Greater than 20' not exceeding 25' |  | Greater than 25' not exceeding 30' |  | Greater than 30' not exceeding 35' |  |
|  | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" | 2 2/3"x1/2" | 3"x1" |
| 12 | (0.075) |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  |
| 15 | (0.075) |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  | (0.075) |  |
| 18 | (0.075) |  | 0.060 |  | 0.060 |  | 0.060 |  | 0.060 |  | (0.075) |  | H 0.060 |  |
| 21 | H 0.060E |  | 0.060 |  | 0.060 |  | 0.060 |  | (0.075) |  | H 0.060 |  | H 0.060E |  |
| 24 | (0.105E) |  | 0.060 |  | 0.060 |  | (0.075) |  | (0.105) |  | (0.105) |  | (0.105E) |  |
| 30 | H 0.075E | H 0.060 | 0.075 | H 0.060 | 0.075 | H 0.060 | (0.105) | H 0.060 | (0.105) | H 0.060 | H 0.075E | H 0.060 | H 0.075E | H 0.060 |
| 36 | (0.135E) | H 0.060E | 0.075 | H 0.060 | (0.105) | H 0.060 | (0.105) | H 0.060 | (0.135) | H 0.060 | H 0.075E | H 0.060 | H 0.075E | H 0.060E |
| 42 | 0.105E | (0.075) | 0.105 | 0.060 | 0.105 | 0.060 | 0.105 | 0.060 | 0.105 | (0.075) | 0.105E | 0.105 | 0.105E | (0.105E) |
| 48 | 0.105E | (0.075) | 0.105 | 0.060 | 0.105 | 0.060 | 0.105 | (0.075) | 0.105 | (0.105) | 0.105E | (0.105E) | 0.105E | (0.135E) |
| 54 | 0.105E | (0.105) | 0.105 | 0.060 | 0.105 | 0.060 | 0.105 | (0.075) | 0.105 | (0.105) | 0.105E | (0.105E) | (0.135E) | (0.135E) |
| 60 | 0.135E | (0.105) | 0.135 | 0.060 | 0.135 | (0.075) | 0.135 | (0.105) | 0.135 | (0.105) | 0.135E | (0.135E) | (0.164E) | (0.135E) |
| 66 | 0.164E | (0.105) | 0.164 | 0.060 | 0.164 | (0.075) | 0.164 | (0.105) | 0.164 | (0.135) | 0.164E | (0.135E) | H 0.164E | (0.135E) |
| 72 | 0.164E | (0.105) | 0.164 | 0.060 | 0.164 | (0.075) | 0.164 | (0.105) | 0.164 | (0.135) | H 0.164E | (0.135E) | H 0.164E | (0.164E) |
| 78 |  | (0.135) |  | 0.075 |  | (0.105) |  | (0.105) |  | (0.135) |  | (0.135E) |  | (0.164E) |
| 84 |  | (0.135) |  | 0.105 |  | 0.105 |  | (0.135) |  | (0.135) |  | (0.164E) |  | (0.164E) |
| 90 |  | (0.135) |  | 0.105 |  | 0.105 |  | (0.135) |  | (0.135) |  | (0.164E) |  | (0.164E) |
| 96 |  | (0.135) |  | 0.105 |  | 0.105 |  | (0.135) |  | (0.135) |  | (0.164E) |  | H 0.135E |
| 102 |  | 0.135Z |  | 0.135 |  | 0.135 |  | 0.135 |  | (0.164) |  | (0.164E) |  | H 0.135E |
| 108 |  | 0.135Z |  | 0.135 |  | 0.135 |  | 0.135 |  | (0.164) |  | (0.164E) |  | H 0.164E |
| 114 |  | 0.164Z |  | 0.164 |  | 0.164 |  | 0.164 |  | 0.164 |  | H 0.164E |  | H 0.164E |
| 120 |  | 0.164Z |  | 0.164 |  | 0.164 |  | 0.164 |  | 0.164 |  | H 0.164E |  |  |

Notes:

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

A thickness preceded by an “H” indicates only helical seam fabrication is allowed.

E	Elongation according to Article 542.04(e), the elongation requirement for Type 1 fill heights may be eliminated for fills above 1'-6"

Z	1”-6” Minimum fill

| TABLE IC: THICKNESS OF CORRUGATED ALUMINUM ALLOY PIPE FOR THE RESPECTIVE DIAMETER OF PIPE AND FILL HEIGHTS OVER THE TOP OF THE PIPE FOR 68 mm x 13 mm AND 75 mm x 25 mm CORRUGATIONS (Metric) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter mm | Type 1 |  | Type 2 |  | Type 3 |  | Type 4 |  | Type 5 |  | Type 6 |  | Type 7 |  |
|  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  | Fill Height: |  |
|  | 1 m and less 0.3 m min. cover |  | Greater than 1 m not exceeding 3 m |  | Greater than 3 m not exceeding 4.5 m |  | Greater than 4.5 m not exceeding 6 m |  | Greater than 6 m not exceeding 7.5 m |  | Greater than 7.5 m not exceeding 9 m |  | Greater than 9 m not exceeding 10.5 m |  |
|  | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm |
| 300 | (1.91) |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  |
| 375 | (1.91) |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  | (1.91) |  |
| 450 | (1.91) |  | 1.52 |  | 1.52 |  | 1.52 |  | 1.52 |  | (1.91) |  | H 1.52 |  |
| 525 | H 1.52E |  | 1.52 |  | 1.52 |  | 1.52 |  | (1.91) |  | H 1.52 |  | H 1.52E |  |
| 600 | (2.67E) |  | 1.52 |  | 1.52 |  | (1.91) |  | (2.67) |  | (2.67) |  | (2.67E) |  |
| 750 | H 1.91E | H 1.52 | 1.91 | H 1.52 | 1.91 | H 1.52 | (2.67) | H 1.52 | (2.67) | H 1.52 | H 1.91E | H 1.52 | H 1.91E | H 1.52 |
| 900 | (3.43E) | H 1.52E | 1.91 | H 1.52 | (2.67) | H 1.52 | (2.67) | H 1.52 | (3.43) | H 1.52 | H 1.91E | H 1.52 | H 1.91E | H 1.52E |
| 1050 | 2.67E | (1.91) | 2.67 | 1.52 | 2.67 | 1.52 | 2.67 | 1.52 | 2.67 | (1.91) | 2.67E | 2.67 | 2.67E | (2.67E) |
| 1200 | 2.67E | (1.91) | 2.67 | 1.52 | 2.67 | 1.52 | 2.67 | (1.91) | 2.67 | (2.67) | 2.67E | (2.67E) | 2.67E | (3.43E) |
| 1350 | 2.67E | (2.67) | 2.67 | 1.52 | 2.67 | 1.52 | 2.67 | (1.91) | 2.67 | (2.67) | 2.67E | (2.67E) | (3.43E) | (3.43E) |
| 1500 | 3.43E | (2.67) | 3.43 | 1.52 | 3.43 | (1.91) | 3.43 | (2.67) | 3.43 | (2.67) | 3.43E | (3.43E) | (4.17E) | (3.43E) |
| 1650 | 4.17E | (2.67) | 4.17 | 1.52 | 4.17 | (1.91) | 4.17 | (2.67) | 4.17 | (3.43) | 4.17E | (3.43E) | H 4.17E | (3.43E) |
| 1800 | 4.17E | (2.67) | 4.17 | 1.52 | 4.17 | (1.91) | 4.17 | (2.67) | 4.17 | (3.43) | H 4.17E | (3.43E) | H 4.17E | (4.17E) |
| 1950 |  | (3.43) |  | 1.91 |  | (2.67) |  | (2.67) |  | (3.43) |  | (3.43E) |  | (4.17E) |
| 2100 |  | (3.43) |  | 2.67 |  | 2.67 |  | (3.43) |  | (3.43) |  | (4.17E) |  | (4.17E) |
| 2250 |  | (3.43) |  | 2.67 |  | 2.67 |  | (3.43) |  | (3.43) |  | (4.17E) |  | (4.17E) |
| 2400 |  | (3.43) |  | 2.67 |  | 2.67 |  | (3.43) |  | (3.43) |  | (4.17E) |  | H 3.43E |
| 2550 |  | 3.43Z |  | 3.43 |  | 3.43 |  | 3.43 |  | (4.17) |  | (4.17E) |  | H 3.43E |
| 2700 |  | 3.43Z |  | 3.43 |  | 3.43 |  | 3.43 |  | (4.17) |  | (4.17E) |  | H 4.17E |
| 2850 |  | 4.17Z |  | 4.17 |  | 4.17 |  | 4.17 |  | 4.17 |  | H 4.17E |  | H 4.17E |
| 3000 |  | 4.17Z |  | 4.17 |  | 4.17 |  | 4.17 |  | 4.17 |  | H 4.17E |  |  |

Notes:

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

A thickness preceded by an “H” indicates only helical seam fabrication is allowed.

E	Elongation according to Article 542.04(e), the elongation requirement for Type 1 fill heights may be eliminated for fills above 450 mm.

Z	450 mm Minimum fill

| Table IIA: THICKNESS FOR CORRUGATED STEEL PIPE ARCHES AND CORRUGATED ALUMINUM ALLOY PIPE ARCHES FOR THE RESPECTIVE EQUIVALENT ROUND SIZE OF PIPE AND FILL HEIGHTS OVER THE TOP OF PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent Round Size in. | Corrugated Steel & Aluminum Pipe Arch 2 2/3" x 1/2" |  | Corrugated Steel & Aluminum Pipe Arch 3" x 1" |  | Corrugated Steel Pipe Arch 5" x 1" |  | Min. Cover | Type 1 |  |  |  |  | Type 2 |  |  |  |  | Type 3 |  |  |  |  |
|  |  |  |  |  |  |  |  | Fill Height: |  |  |  |  | Fill Height: |  |  |  |  | Fill Height: |  |  |  |  |
|  |  |  |  |  |  |  |  | 3' and less |  |  |  |  | Greater than 3' not exceeding 10' |  |  |  |  | Greater than 10' not exceeding 15' |  |  |  |  |
|  | Span (in.)* | Rise (in.) | Span (in.) | Rise (in.) | Span (in.) | Rise (in.) | Steel & Aluminum | Steel |  |  | Aluminum |  | Steel |  |  | Aluminum |  | Steel |  |  | Aluminum |  |
|  |  |  |  |  |  |  |  | 2 2/3" x 1/2" | 3"x1" | 5" x 1" | 2 2/3" x 1/2" | 3"x1" | 2 2/3" x 1/2" | 3"x1" | 5" x 1" | 2 2/3" x 1/2" | 3"x1" | 2 2/3" x 1/2" | 3"x1" | 5" x 1" | 2 2/3" x 1/2" | 3"x1" |
| 15 | 17 | 13 |  |  |  |  | 1'-6" | 0.064 |  |  | 0.060 |  | 0.064 |  |  | 0.060 |  | 0.064 |  |  | 0.060 |  |
| 18 | 21 | 15 |  |  |  |  | 1'-6" | 0.064 |  |  | 0.060 |  | 0.064 |  |  | 0.060 |  | 0.064 |  |  | 0.060 |  |
| 21 | 24 | 18 |  |  |  |  | 1'-6" | 0.064 |  |  | (0.075) |  | 0.064 |  |  | 0.060 |  | 0.064 |  |  | 0.060 |  |
| 24 | 28 | 20 |  |  |  |  | 1'-6" | (0.079) |  |  | (0.105) |  | 0.064 |  |  | 0.075 |  | 0.064 |  |  | 0.075 |  |
| 30 | 35 | 24 |  |  |  |  | 1'-6" | (0.079) |  |  | (0.105) |  | 0.064 |  |  | 0.075 |  | (0.079) |  |  | (0.105) |  |
| 36 | 42 | 29 |  |  |  |  | 1'-6" | (0.079) |  |  | 0.105 |  | 0.064 |  |  | 0.105 |  | 0.064 |  |  | 0.105 |  |
| 42 | 49 | 33 |  |  |  |  | 1'-6" | 0.109 |  |  | 0.105 |  | (0.109) |  |  | 0.105 |  | (0.109) |  |  | 0.105 |  |
| 48 | 57 | 38 | 53 | 41 | 53 | 41 | 1'-6" | 0.109 | (0.109) | (0.109) | 0.135 | 0.060 | 0.109 | 0.079 | 0.079 | 0.135 | 0.060 | 0.109 | 0.079 | (0.109) | 0.135 | 0.060 |
| 54 | 64 | 43 | 60 | 46 | 60 | 46 | 1'-6" | 0.109 | (0.109) | 0.109 | 0.164 | (0.075) | 0.109 | 0.079 | 0.079 | 0.164 | 0.060 | 0.109 | (0.109) | 0.109 | 0.164 | (0.075) |
| 60 | 71 | 47 | 66 | 51 | 66 | 51 | 1'-6" | 0.138 | (0.109) | 0.109 | 0.164 | (0.075) | 0.138 | 0.079 | (0.109) | 0.164 | 0.060 | 0.138 | (0.109) | 0.109 | 0.164 | (0.075) |
| 66 | 77 | 52 | 73 | 55 | 73 | 55 | 1'-6" | 0.168 | (0.109) | 0.109 |  | 0.075 | 0.168 | 0.079 | (0.109) |  | 0.075 | 0.168 | (0.109) | 0.109 |  | 0.075 |
| 72 | 83 | 57 | 81 | 59 | 81 | 59 | 1'-6" | 0.168 | (0.109) | 0.109 |  | 0.105 | 0.168 | 0.079 | (0.109) |  | 0.105 | 0.168 | (0.109) | 0.109 |  | 0.105 |
| 78 |  |  | 87 | 63 | 87 | 63 | 1'-6" |  | 0.109 | 0.109 |  | 0.105 |  | (0.109) | 0.109 |  | 0.105 |  | 0.109 | 0.109 |  | 0.105 |
| 84 |  |  | 95 | 67 | 95 | 67 | 1'-6" |  | 0.109 | 0.109 |  | 0.105 |  | (0.109) | 0.109 |  | 0.105 |  | 0.109 | 0.109 |  | 0.105 |
| 90 |  |  | 103 | 71 | 103 | 71 | 1'-6" |  | 0.109 | 0.109 |  | 0.135 |  | (0.109) | 0.109 |  | 0.135 |  | 0.109 | 0.109 |  | 0.135 |
| 96 |  |  | 112 | 75 | 112 | 75 | 1'-6" |  | 0.109 | (0.138) |  | 0.164 |  | 0.109 | 0.109 |  | 0.164 |  | 0.109 | (0.138) |  | 0.164 |
| 102 |  |  | 117 | 79 | 117 | 79 | 1'-6" |  | 0.109 | (0.138) |  | 0.164 |  | 0.109 | 0.109 |  | 0.164 |  | 0.109 | (0.138) |  | 0.164 |
| 108 |  |  | 128 | 83 | 128 | 83 | 1'-6" |  | 0.138 | 0.138 |  |  |  | 0.138 | 0.138 |  |  |  | 0.138 | 0.138 |  |  |
| 114 |  |  | 137 | 87 | 137 | 87 | 1'-6" |  | 0.138 | 0.138 |  |  |  | 0.138 | 0.138 |  |  |  | 0.138 | 0.138 |  |  |
| 120 |  |  | 142 | 91 | 142 | 91 | 1'-6" |  | 0.168 | 0.168 |  |  |  | 0.168 | 0.168 |  |  |  | 0.168 | 0.168 |  |  |

Notes:

*	Aluminized Type 2 Steel or Precoated Galvanized Steel shall be required for steel spans up to 42” according to Article 1006.01.

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

The Type 1 corrugated steel or aluminum pipe arches shall be placed on soil having a minimum bearing capacity of 3 tons per square foot.

The Type 2 and 3 corrugated steel or aluminum pipe arches shall be placed on soil having a minimum bearing capacity of 2 tons per square foot.

This minimum bearing capacity will be determined by the Engineer in the field.

| Table IIA: THICKNESS FOR CORRUGATED STEEL PIPE ARCHES AND CORRUGATED ALUMINUM ALLOY PIPE ARCHES FOR THE RESPECTIVE EQUIVALENT ROUND SIZE OF PIPE AND FILL HEIGHTS OVER THE TOP OF PIPE (Metric) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent Round Size (mm) | Corrugated Steel & Aluminum Pipe Arch 68 x 13 mm |  | Corrugated Steel & Aluminum Pipe Arch 75 x 25 mm |  | Corrugated Steel Pipe Arch 125 x 25 mm |  | Min. Cover | Type 1 |  |  |  |  | Type 2 |  |  |  |  | Type 3 |  |  |  |  |
|  |  |  |  |  |  |  |  | Fill Height: |  |  |  |  | Fill Height: |  |  |  |  | Fill Height: |  |  |  |  |
|  |  |  |  |  |  |  |  | 1 m and less |  |  |  |  | Greater than 1 m not exceeding 3 m |  |  |  |  | Greater than 3 m not exceeding 4.5 m |  |  |  |  |
|  | Span (mm)* | Rise (mm) | Span (mm) | Rise (mm) | Span (mm) | Rise (mm) | Steel & Aluminum | Steel |  |  | Aluminum |  | Steel |  |  | Aluminum |  | Steel |  |  | Aluminum |  |
|  |  |  |  |  |  |  |  | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm | 68 x 13 mm | 75 x 25 mm | 125 x 25 mm | 68 x 13 mm | 75 x 25 mm |
| 375 | 430 | 330 |  |  |  |  | 0.5 m | 1.63 |  |  | 1.52 |  | 1.63 |  |  | 1.52 |  | 1.63 |  |  | 1.52 |  |
| 450 | 530 | 380 |  |  |  |  | 0.5 m | 1.63 |  |  | 1.52 |  | 1.63 |  |  | 1.52 |  | 1.63 |  |  | 1.52 |  |
| 525 | 610 | 460 |  |  |  |  | 0.5 m | 1.63 |  |  | (1.91) |  | 1.63 |  |  | 1.52 |  | 1.63 |  |  | 1.52 |  |
| 600 | 710 | 510 |  |  |  |  | 0.5 m | (2.01) |  |  | (2.67) |  | 1.63 |  |  | 1.91 |  | 1.63 |  |  | 1.91 |  |
| 750 | 870 | 630 |  |  |  |  | 0.5 m | (2.01) |  |  | (2.67) |  | 1.63 |  |  | 1.91 |  | (2.01) |  |  | (2.67) |  |
| 900 | 1060 | 740 |  |  |  |  | 0.5 m | (2.01) |  |  | 2.67 |  | 1.63 |  |  | 2.67 |  | 1.63 |  |  | 2.67 |  |
| 1050 | 1240 | 840 |  |  |  |  | 0.5 m | 2.77 |  |  | 2.67 |  | (2.77) |  |  | 2.67 |  | (2.77) |  |  | 2.67 |  |
| 1200 | 1440 | 970 | 1340 | 1050 | 1340 | 1050 | 0.5 m | 2.77 | (2.77) | (2.77) | 3.43 | 1.52 | 2.77 | 2.01 | 2.01 | 3.43 | 1.52 | 2.77 | 2.01 | (2.77) | 3.43 | 1.52 |
| 1350 | 1620 | 1100 | 1520 | 1170 | 1520 | 1170 | 0.5 m | 2.77 | (2.77) | 2.77 | 4.17 | (1.91) | 2.77 | 2.01 | 2.01 | 4.17 | 1.52 | 2.77 | (2.77) | 2.77 | 4.17 | (1.91) |
| 1500 | 1800 | 1200 | 1670 | 1300 | 1670 | 1300 | 0.5 m | 3.51 | (2.77) | 2.77 | 4.17 | (1.91) | 3.51 | 2.01 | (2.77) | 4.17 | 1.52 | 3.51 | (2.77) | 2.77 | 4.17 | (1.91) |
| 1650 | 1950 | 1320 | 1850 | 1400 | 1850 | 1400 | 0.5 m | 4.27 | (2.77) | 2.77 |  | 1.91 | 4.27 | 2.01 | (2.77) |  | 1.91 | 4.27 | (2.77) | 2.77 |  | 1.91 |
| 1800 | 2100 | 1450 | 2050 | 1500 | 2050 | 1500 | 0.5 m | 4.27 | (2.77) | 2.77 |  | 2.67 | 4.27 | 2.01 | (2.77) |  | 2.67 | 4.27 | (2.77) | 2.77 |  | 2.67 |
| 1950 |  |  | 2200 | 1620 | 2200 | 1620 | 0.5 m |  | 2.77 | 2.77 |  | 2.67 |  | (2.77) | 2.77 |  | 2.67 |  | 2.77 | 2.77 |  | 2.67 |
| 2100 |  |  | 2400 | 1720 | 2400 | 1720 | 0.5 m |  | 2.77 | 2.77 |  | 2.67 |  | (2.77) | 2.77 |  | 2.67 |  | 2.77 | 2.77 |  | 2.67 |
| 2250 |  |  | 2600 | 1820 | 2600 | 1820 | 0.5 m |  | 2.77 | 2.77 |  | 3.43 |  | (2.77) | 2.77 |  | 3.43 |  | 2.77 | 2.77 |  | 3.43 |
| 2400 |  |  | 2840 | 1920 | 2840 | 1920 | 0.5 m |  | 2.77 | (3.51) |  | 4.17 |  | 2.77 | 2.77 |  | 4.17 |  | 2.77 | (3.51) |  | 4.17 |
| 2550 |  |  | 2970 | 2020 | 2970 | 2020 | 0.5 m |  | 2.77 | (3.51) |  | 4.17 |  | 2.77 | 2.77 |  | 4.17 |  | 2.77 | (3.51) |  | 4.17 |
| 2700 |  |  | 3240 | 2120 | 3240 | 2120 | 0.5 m |  | 3.51 | 3.51 |  |  |  | 3.51 | 3.51 |  |  |  | 3.51 | 3.51 |  |  |
| 2850 |  |  | 3470 | 2220 | 3470 | 2220 | 0.5 m |  | 3.51 | 3.51 |  |  |  | 3.51 | 3.51 |  |  |  | 3.51 | 3.51 |  |  |
| 3000 |  |  | 3600 | 2320 | 3600 | 2320 | 0.5 m |  | 4.27 | 4.27 |  |  |  | 4.27 | 4.27 |  |  |  | 4.27 | 4.27 |  |  |

Notes:

*	Aluminized Type 2 Steel or Precoated Galvanized Steel shall be required for steel spans up to 1060 mm according to Article 1006.01.

Thicknesses are based on longitudinal riveted seam fabrication, values in “()” can be reduced by one gage thickness if helical seam fabrication is utilized.

The Type 1 corrugated steel or aluminum pipe arches shall be placed on soil having a minimum bearing capacity of 290 kN per square meter.

The Type 2 and 3 corrugated steel or aluminum pipe arches shall be placed on soil having a minimum bearing capacity of 192 kN per square meter.

This minimum bearing capacity will be determined by the Engineer in the field.

| Table IIB: CLASSES OF REINFORCED CONCRETE ELLIPTICALL AND REINFORCED CONCRETE ARCH PIPE FOR THE RESPECTIVE EQUIVALENT ROUND SIZE OF PIPE AND FILL HEIGHTS OVER THE TOP OF PIPE |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent Round Size (in.) | Reinforced Concrete Elliptical pipe (in.) |  | Reinforced Concrete Arch pipe (in.) |  | Minimum Cover | Type 1 |  | Type 2 |  | Type 3 |  |
|  |  |  |  |  |  | Fill Height: 3' and less |  | Fill Height: Greater than 3' not exceeding 10' |  | Fill Height: Greater than 10' not exceeding 15' |  |
|  | Span | Rise | Span | Rise | RCCP HE & A | HE | Arch | HE | Arch | HE | Arch |
| 15 | 23 | 14 | 18 | 11 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 18 | 23 | 14 | 22 | 13 1/2 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 21 | 30 | 19 | 26 | 15 1/2 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 24 | 30 | 19 | 28 1/2 | 18 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 27 | 34 | 22 | 36 1/4 | 22 1/2 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 30 | 38 | 24 | 36 1/4 | 22 1/2 | 1' -0" | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 36 | 45 | 29 | 43 3/4 | 26 5/8 | 1' -0" | HE-II | A-II | HE-III | A-III | HE-IV | A-IV |
| 42 | 53 | 34 | 51 1/8 | 31 5/16 | 1' -0" | HE-I | A-II | HE-III | A-III | HE-IV | A-IV |
| 48 | 60 | 38 | 58 1/2 | 36 | 1' -0" | HE-I | A-II | HE-III | A-III | 1460 | 1450 |
| 54 | 68 | 43 | 65 | 40 | 1' -0" | HE-I | A-II | HE-III | A-III | 1460 | 1460 |
| 60 | 76 | 48 | 73 | 45 | 1' -0" | HE-I | A-II | HE-III | A-III | 1460 | 1470 |
| 66 | 83 | 53 | 88 | 54 | 1' -0" | HE-I | A-II | HE-III | A-III | 1470 | 1480 |
| 72 | 91 | 58 | 88 | 54 | 1' -0" | HE-I | A-II | HE-III | A-III | 1470 | 1480 |

Notes:

A number indicates the D-Load for the diameter and depth of fill and that a special design is required.

Design assumptions; Water filled pipe, AASHTO Type 2 installation per AASHTO LRFD Table 12.10.2.1-1

| Table IIB: CLASSES OF REINFORCED CONCRETE ELLIPTICALL AND REINFORCED CONCRETE ARCH PIPE FOR THE RESPECTIVE EQUIVALENT ROUND SIZE OF PIPE AND FILL HEIGHTS OVER THE TOP OF PIPE (Metric) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equivalent Round Size (mm) | Reinforced Concrete Elliptical pipe (mm) |  | Reinforced Concrete Arch pipe (mm) |  | Minimum Cover | Type 1 |  | Type 2 |  | Type 3 |  |
|  |  |  |  |  |  | Fill Height: 1 m and less |  | Fill Height: Greater than 1 m not exceeding 3 m |  | Fill Height: Greater than 3 m not exceeding 4.5 m |  |
|  | Span | Rise | Span | Rise | RCCP HE & A | HE | Arch | HE | Arch | HE | Arch |
| 375 | 584 | 356 | 457 | 279 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 450 | 584 | 356 | 559 | 343 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 525 | 762 | 483 | 660 | 394 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 600 | 762 | 483 | 724 | 457 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 686 | 864 | 559 | 921 | 572 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 750 | 965 | 610 | 921 | 572 | 0.3 m | HE-III | A-III | HE-III | A-III | HE-IV | A-IV |
| 900 | 1143 | 737 | 1111 | 676 | 0.3 m | HE-II | A-II | HE-III | A-III | HE-IV | A-IV |
| 1050 | 1346 | 864 | 1299 | 795 | 0.3 m | HE-I | A-II | HE-III | A-III | HE-IV | A-IV |
| 1200 | 1524 | 965 | 1486 | 914 | 0.3 m | HE-I | A-II | HE-III | A-III | 70 | 70 |
| 1350 | 1727 | 1092 | 1651 | 1016 | 0.3 m | HE-I | A-II | HE-III | A-III | 70 | 70 |
| 1500 | 1930 | 1219 | 1854 | 1143 | 0.3 m | HE-I | A-II | HE-III | A-III | 70 | 70 |
| 1676 | 2108 | 1346 | 2235 | 1372 | 0.3 m | HE-I | A-II | HE-III | A-III | 70 | 70 |
| 1800 | 2311 | 1473 | 2235 | 1372 | 0.3 m | HE-I | A-II | HE-III | A-III | 70 | 70 |

Notes:

A number indicates the D-Load for the diameter and depth of fill and that a special design is required.

Design assumptions; Water filled pipe, AASHTO Type 2 installation per AASHTO LRFD Table 12.10.2.1-1

| TABLE IIIA: PLASTIC PIPE PERMITTED FOR A GIVEN PIPE DIAMETER AND FILL HEIGHT OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter (in.) | Type 1 |  |  |  |  | Type 2 |  |  |  |  | Type 3 |  |  |  |  | Type 4 |  |  |  |
|  | Fill Height: 3' and less, with 1' min |  |  |  |  | Fill Height: Greater than 3', not exceeding 10' |  |  |  |  | Fill Height: Greater than 10', not exceeding 15' |  |  |  |  | Fill Height: Greater than 15', not exceeding 20' |  |  |  |
|  | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPP |
| 10 | X | X | X | X | NA | X | X | X | X | NA | X | X | X | X | NA | X | X | X | NA |
| 12 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 15 | X | X | NA | X | X | X | X | NA | X | X | X | X | NA | NA | X | X | X | NA | X |
| 18 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 21 | X | X | NA | NA | NA | X | X | NA | NA | NA | X | X | NA | NA | NA | X | X | NA | NA |
| 24 | X | X | X | X | X | X | X | X | X | X | X | X | NA | NA | NA | X | X | X | NA |
| 30 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 36 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | NA | X | X | X | NA |
| 42 | X | NA | X | X | NA | X | NA | X | NA | NA | X | NA | X | NA | NA | X | NA | X | NA |
| 48 | X | NA | X | X | X | X | NA | X | NA | NA | X | NA | X | NA | NA | X | NA | X | NA |

Notes:

PVC	Polyvinyl Chloride (PVC) pipe with a smooth interior

CPVC	Corrugated Polyvinyl Chloride (CPVC) pipe with a smooth interior

PE	Polyethylene (PE) pipe with a smooth interior

CPE	Corrugated Polyethylene (PE) pipe with a smooth interior

CPP	Corrugated Polypropylene (CPP) pipe with a smooth interior

X	This material may be used for the given pipe diameter and fill height

NA 	Not Available

| TABLE IIIA: PLASTIC PIPE PERMITTED FOR A GIVEN PIPE DIAMETER AND FILL HEIGHT OVER THE TOP OF THE PIPE (Metric) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter (mm) | Type 1 |  |  |  |  | Type 2 |  |  |  |  | Type 3 |  |  |  |  | Type 4 |  |  |  |
|  | Fill Height: 1 m and less, with 0.3 m min. cover |  |  |  |  | Fill Height: Greater than 1 m, not exceeding 3 m |  |  |  |  | Fill Height: Greater than 3 m, not exceeding 4.5 m |  |  |  |  | Fill Height: Greater than 4.5 m, not exceeding 6 m |  |  |  |
|  | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPE | CPP | PVC | CPVC | PE | CPP |
| 250 | X | X | X | X | NA | X | X | X | X | NA | X | X | X | X | NA | X | X | X | NA |
| 300 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 375 | X | X | NA | X | X | X | X | NA | X | X | X | X | NA | NA | X | X | X | NA | X |
| 450 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 525 | X | X | NA | NA | NA | X | X | NA | NA | NA | X | X | NA | NA | NA | X | X | NA | NA |
| 600 | X | X | X | X | X | X | X | X | X | X | X | X | NA | NA | NA | X | X | X | NA |
| 750 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | X | X | X | X | NA |
| 900 | X | X | X | X | X | X | X | X | X | X | X | X | X | NA | NA | X | X | X | NA |
| 1000 | X | NA | X | X | NA | X | NA | X | NA | NA | X | NA | X | NA | NA | X | NA | X | NA |
| 1200 | X | NA | X | X | X | X | NA | X | NA | NA | X | NA | X | NA | NA | X | NA | X | NA |

Notes:

PVC	Polyvinyl Chloride (PVC) pipe with a smooth interior

CPVC	Corrugated Polyvinyl Chloride (CPVC) pipe with a smooth interior

PE	Polyethylene (PE) pipe with a smooth interior

CPE	Corrugated Polyethylene (PE) pipe with a smooth interior

CPP	Corrugated Polypropylene (CPP) pipe with a smooth interior

X	This material may be used for the given pipe diameter and fill height

NA 	Not Available

| TABLE IIIB: PLASTIC PIPE PERMITTED |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOR A GIVEN PIPE DIAMETER AND FILL HEIGHT OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal Diameter (in.) | Type 5 |  |  |  | Type 6 |  |  |  | Type 7 |  |  |  |
|  | Fill Height: Greater than 20', not exceeding 25' |  |  |  | Fill Height: Greater than 25', not exceeding 30' |  |  |  | Fill Height: Greater than 30', not exceeding 35' |  |  |  |
|  | PVC | CPVC |  |  | PVC | CPVC |  |  | CPVC |  |  |  |
| 10 | X | X |  |  | X | X |  |  | X |  |  |  |
| 12 | X | X |  |  | X | X |  |  | X |  |  |  |
| 15 | X | X |  |  | X | X |  |  | X |  |  |  |
| 18 | X | X |  |  | X | X |  |  | X |  |  |  |
| 21 | X | X |  |  | X | X |  |  | X |  |  |  |
| 24 | X | X |  |  | X | X |  |  | X |  |  |  |
| 30 | X | X |  |  | X | X |  |  | X |  |  |  |
| 36 | X | X |  |  | X | X |  |  | X |  |  |  |
| 42 | X | NA |  |  | X | NA |  |  | NA |  |  |  |
| 48 | X | NA |  |  | X | NA |  |  | NA |  |  |  |

Notes:

PVC	Polyvinyl Chloride (PVC) pipe with a smooth interior

CPVC	Corrugated Polyvinyl Chloride (CPVC) pipe with a smooth interior

X	This material may be used for the given pipe diameter and fill height

NA 	Not Available

| TABLE IIIB: PLASTIC PIPE PERMITTED |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOR A GIVEN PIPE DIAMETER AND FILL HEIGHT OVER THE TOP OF THE PIPE (metric) |  |  |  |  |  |  |  |  |  |  |  |  |
| Nominal Diameter (mm) | Type 5 |  |  |  | Type 6 |  |  |  | Type 7 |  |  |  |
|  | Fill Height: Greater than 6 m, not exceeding 7.5 m |  |  |  | Fill Height: Greater than 7.5 m, not exceeding 9 m |  |  |  | Fill Height: Greater than 9 m, not exceeding 10.5 m |  |  |  |
|  | PVC | CPVC |  |  | PVC | CPVC |  |  | CPVC |  |  |  |
| 250 | X | X |  |  | X | X |  |  | X |  |  |  |
| 300 | X | X |  |  | X | X |  |  | X |  |  |  |
| 375 | X | X |  |  | X | X |  |  | X |  |  |  |
| 450 | X | X |  |  | X | X |  |  | X |  |  |  |
| 525 | X | X |  |  | X | X |  |  | X |  |  |  |
| 600 | X | X |  |  | X | X |  |  | X |  |  |  |
| 750 | X | X |  |  | X | X |  |  | X |  |  |  |
| 900 | X | X |  |  | X | X |  |  | X |  |  |  |
| 1000 | X | NA |  |  | X | NA |  |  | NA |  |  |  |
| 1200 | X | NA |  |  | X | NA |  |  | NA |  |  |  |

Notes:

PVC	Polyvinyl Chloride (PVC) pipe with a smooth interior

CPVC	Corrugated Polyvinyl Chloride (CPVC) pipe with a smooth interior

PE	Polyethylene (PE) pipe with a smooth interior

X	This material may be used for the given pipe diameter and fill height

NA 	Not Available”

Revise the first sentence of the first paragraph of Article 542.04(c) of the Standard Specifications to read:

“	Compacted aggregate, at least 4 in. (100 mm) in depth below the pipe culvert, shall be placed the entire width of the trench and for the length of the pipe culvert, except compacted impervious material shall be used for the outer 3 ft (1 m) at each end of the pipe culvert.”

Revise the seventh paragraph of Article 542.04(d) of the Standard Specifications to read:

“PVC, PE and CPP pipes shall be joined according to the manufacturer’s specifications.”

Replace the third sentence of the first paragraph of Article 542.04(h) of the Standard Specifications with the following:

“	The total cover required for various construction loadings shall be the responsibility of the Contractor.”

Delete “Table IV : Wheel Loads and Total Cover” in Article 542.04(h) of the Standard Specifications.

Revise the first and second paragraphs of Article 542.04(i) of the Standard Specifications to read:

“	(i)	Deflection Testing for Pipe Culverts.  All PE, PVC and CPP pipe culverts shall be tested for deflection not less than 30 days after the pipe is installed and the backfill compacted.  The testing shall be performed in the presence of the Engineer.

For PVC, PE, and CPP pipe culverts with diameters 24 in. (600 mm) or smaller, a mandrel drag shall be used for deflection testing.  For PVC, PE, and CPP pipe culverts with diameters over 24 in. (600 mm), deflection measurements other than by a mandrel shall be used.”

Revise Articles 542.04(i)(1) and (2) of the Standard Specifications to read:

“	(1)	For all PVC pipe: as defined using ASTM D 3034 methodology.

(2)	For all PE and CPP pipe:  the average inside diameter based on the minimum and maximum tolerances specified in the corresponding ASTM or AASHTO material specifications.”

Revise the second sentence of the second paragraph of Article 542.07 of the Standard Specifications to read:

“	When a prefabricated end section is used, it shall be of the same material as the pipe culvert, except for polyethylene (PE), polyvinylchloride (PVC), and polypropylene (PP) pipes which shall have metal end sections.”

Revise the first paragraph of Article 1040.03 of the Standard Specifications to read:

“	1040.03	Polyvinyl Chloride (PVC) Pipe.  Acceptance testing of PVC pipe and fittings shall be accomplished during the same construction season in which they are installed.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties.  The pipe shall meet the following additional requirements.”

Delete Articles 1040.03(e) and (f) of the Standard Specifications.

Revise Articles 1040.04(c) and (d) of the Standard Specifications to read:

“	(c)	PE Profile Wall Pipe for Insertion Lining.  The pipe shall be according to ASTM F 894.  When used for insertion lining of pipe culverts, the pipe liner shall have a minimum pipe stiffness of 46 psi (317 kPa) at five percent deflection for nominal inside diameters of 42 in. (1050 mm) or less.  For nominal inside diameters of greater than 42 in. (1050 mm), the pipe liner shall have a minimum pipe stiffness of 32.5 psi (225 kPa) at five percent deflection.  All sizes shall have wall construction that presents essentially smooth internal and external surfaces.

(d)	PE Pipe with a Smooth Interior.  The pipe shall be according to ASTM F 714 (DR 32.5) with a minimum cell classification of PE 335434 as defined in ASTM D 3350.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties and the resin used to manufacture the pipe meets or exceeds the minimum cell classification requirements.”

Add the following to Section 1040 of the Standard Specifications:

“	1040.08	Polypropylene (PP) Pipe.  Storage and handling shall be according to the manufacturer's recommendations, except in no case shall the pipe be exposed to direct sunlight for more than six months.  Acceptance testing of the pipe shall be accomplished during the same construction season in which it is installed.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties.  The pipe shall meet the following additional requirements.

(a)	Corrugated PP Pipe with a Smooth Interior.  The pipe shall be according to AAHSTO M 330 (nominal size – 12 to 60 in. (300 to 1500 mm)).  The pipe shall be Type S or D.

(b)	Perforated Corrugated PP Pipe with A Smooth Interior.  The pipe shall be according to AASHTO M 330 (nominal size – 12 to 60 in. (300 to 1500 mm)).  The pipe shall be Type SP.  In addition, the top centerline of the pipe shall be marked so that it is readily visible from the top of the trench before backfilling, and the upper ends of the slot perforations shall be a minimum of ten degrees below the horizontal.”

80324