All Regional Engineers

Omer M. Osman, P.E.

Special Provision for LRFD Storm Sewer Burial Tables

July 25, 2014

This special provision was developed by the Bureau of Bridges and Structures as a result of updating the storm sewer pipe burial tables to be compliant with the AASHTO LRFD Design Code along with updating the pipe materials available.

Highlights of the changes include:

- LRFD compliance for all tables and materials.

- Upgraded to 75 year design life.

- Added CPP flexible pipe material.

- Removed profile wall PVC and profile wall PE flexible pipe materials.

- Added D loads for special design concrete pipe to the storm sewer tables.

This special provision has been revised to correct a few minor rounding errors for various RCCP and CPE storm sewer options.

This special provision should be inserted into contracts involving storm sewer installation.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 7, 2014 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory July 25, 2014.

80325m

# lrfd storm sewer burial tables (bde)

Effective:  November 1, 2013

Revised:  November 1, 2014

Revise Article 550.02 of the Standard Specifications to read as follows:

“Item	Article Section

(a)	Clay Sewer Pipe 	1040.02

(b)	Extra Strength Clay Pipe 	1040.02

(c)	Concrete Sewer, Storm Drain, and Culvert Pipe 	1042

(d)	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe 	1042

(e)	Reinforced Concrete Elliptical Culvert, Storm Drain,and Sewer Pipe (Note 1) 	1042

(f)	Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe (Note 1) 	1042

(g)	Polyvinyl Chloride (PVC) Pipe 	1040.03

(h)	Corrugated Polyvinyl Chloride (PVC) Pipe with a Smooth Interior 	1040.03

(i)	Corrugated Polypropylene (CPP) Pipe with Smooth Interior 	1040.07

(j)	Rubber Gaskets and Preformed Flexible Joint Sealants for Concrete Pipe 	1056

(k)	Mastic Joint Sealer for Pipe 	1055

(l)	External Sealing Band 	1057

(m)	Fine Aggregate (Note 2) 	1003.04

(n)	Coarse Aggregate (Note 3) 	1004.05

(o)	Reinforcement Bars and Welded Wire Fabric 	1006.10

(p)	Handling Hole Plugs 	1042.16

(q)	Polyethylene (PE) Pipe with a Smooth Interior 	1040.04

(r)	Corrugated Polyethylene (PE) Pipe with a Smooth Interior 	1040.04

Note 1.  The class of elliptical and arch pipe used for various storm sewer sizes and heights of fill shall conform to the requirements for circular pipe.

Note 2.  The fine aggregate shall be moist.

Note 3.  The coarse aggregate shall be wet.”

Revise the table for permitted materials in Article 550.03 of the Standard Specifications as follows:

| “Class | #### Materials |
| --- | --- |
| A | Rigid Pipes: |
|  | Clay Sewer Pipe |
|  | Extra Strength Clay Pipe |
|  | Concrete Sewer, Storm Drain, and Culvert Pipe |
|  | Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe |
| B | Rigid Pipes: |
|  | Clay Sewer Pipe |
|  | Extra Strength Clay Pipe |
|  | Concrete Sewer, Storm Drain, and Culvert Pipe |
|  | Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Elliptical Culvert, Storm Drain, and Sewer Pipe |
|  | Reinforced Concrete Arch Culvert, Storm Drain, and Sewer Pipe |
|  | Flexible Pipes: |
|  | Polyvinyl Chloride (PVC) Pipe |
|  | Corrugated Polyvinyl Chloride Pipe (PVC) with a Smooth Interior |
|  | Polyethylene (PE) Pipe with a Smooth Interior Corrugated Polyethylene (PE) Pipe with a Smooth Interior Corrugated Polypropylene (CPP) Pipe with a Smooth Interior” |

Replace the storm sewers tables in Article 550.03 of the Standard Specifications with the following:

| STORM SEWERS KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 1 |  |  |  |  |  |  |  | Type 2 |  |  |  |  |  |  |  |
|  | Fill Height: 3' and less With 1' minimum cover |  |  |  |  |  |  |  | Fill Height: Greater than 3' not exceeding 10' |  |  |  |  |  |  |  |
|  | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP |
| 10 | NA | 3 | X | X | X | X | X | NA | NA | 1 | *X | X | X | X | X | NA |
| 12 | IV | NA | X | X | X | X | X | X | II | 1 | *X | X | X | X | X | X |
| 15 | IV | NA | NA | X | X | NA | X | X | II | 1 | *X | X | X | NA | X | X |
| 18 | IV | NA | NA | X | X | X | X | X | II | 2 | X | X | X | X | X | X |
| 21 | III | NA | NA | X | X | NA | NA | NA | II | 2 | X | X | X | NA | NA | NA |
| 24 | III | NA | NA | X | X | X | X | X | II | 2 | X | X | X | X | X | X |
| 27 | III | NA | NA | NA | NA | NA | NA | NA | II | 3 | X | NA | NA | NA | NA | NA |
| 30 | IV | NA | NA | X | X | X | X | X | II | 3 | X | X | X | X | X | X |
| 33 | III | NA | NA | NA | NA | NA | NA | NA | II | NA | X | NA | NA | NA | NA | NA |
| 36 | III | NA | NA | X | X | X | X | X | II | NA | X | X | X | X | X | X |
| 42 | II | NA | X | X | NA | X | X | NA | II | NA | X | X | NA | X | NA | NA |
| 48 | II | NA | X | X | NA | X | X | X | II | NA | X | X | NA | X | NA | NA |
| 54 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 60 | II | NA | NA | NA | NA | NA | NA | X | II | NA | NA | NA | NA | NA | NA | X |
| 66 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 72 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 78 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 84 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 90 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 96 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |
| 102 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |
| 108 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

CSP	Concrete Sewer, Storm drain, and Culvert Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

PE	Polyethylene Pipe with a Smooth Interior

CPE	Corrugated Polyethylene Pipe with a Smooth Interior

CPP	Corrugated Polypropylene pipe with a Smooth Interior

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

*	May also use Standard Strength Clay Pipe

| STORM SEWERS (Metric) KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 1 |  |  |  |  |  |  |  | Type 2 |  |  |  |  |  |  |  |
|  | Fill Height: 1 m and less With 300 mm minimum cover |  |  |  |  |  |  |  | Fill Height: Greater than 1 m not exceeding 3 m |  |  |  |  |  |  |  |
|  | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP |
| 250 | NA | 3 | X | X | X | X | X | NA | NA | 1 | *X | X | X | X | X | NA |
| 300 | IV | NA | X | X | X | X | X | X | II | 1 | *X | X | X | X | X | X |
| 375 | IV | NA | NA | X | X | NA | X | X | II | 1 | *X | X | X | NA | X | X |
| 450 | IV | NA | NA | X | X | X | X | X | II | 2 | X | X | X | X | X | X |
| 525 | III | NA | NA | X | X | NA | NA | NA | II | 2 | X | X | X | NA | NA | NA |
| 600 | III | NA | NA | X | X | X | X | X | II | 2 | X | X | X | X | X | X |
| 675 | III | NA | NA | NA | NA | NA | NA | NA | II | 3 | X | NA | NA | NA | NA | NA |
| 750 | IV | NA | NA | X | X | X | X | X | II | 3 | X | X | X | X | X | X |
| 825 | III | NA | NA | NA | NA | NA | NA | NA | II | NA | X | NA | NA | NA | NA | NA |
| 900 | III | NA | NA | X | X | X | X | X | II | NA | X | X | X | X | X | X |
| 1050 | II | NA | X | X | NA | X | X | NA | II | NA | X | X | NA | X | NA | NA |
| 1200 | II | NA | X | X | NA | X | X | X | II | NA | X | X | NA | X | NA | NA |
| 1350 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 1500 | II | NA | NA | NA | NA | NA | NA | X | II | NA | NA | NA | NA | NA | NA | X |
| 1650 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 1800 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 1950 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 2100 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 2250 | II | NA | NA | NA | NA | NA | NA | NA | II | NA | NA | NA | NA | NA | NA | NA |
| 2400 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |
| 2550 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |
| 2700 | II | NA | NA | NA | NA | NA | NA | NA | III | NA | NA | NA | NA | NA | NA | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

CSP	Concrete Sewer, Storm drain, and Culvert Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

PE	Polyethylene Pipe with a Smooth Interior

CPE	Corrugated Polyethylene Pipe with a Smooth Interior

CPP	Corrugated Polypropylene pipe with a Smooth Interior

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

*	May also use Standard Strength Clay Pipe

| STORM SEWERS KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 3 |  |  |  |  |  |  |  | Type 4 |  |  |  |  |  |  |
|  | Fill Height: Greater than 10' not exceeding 15' |  |  |  |  |  |  |  | Fill Height: Greater than 15' not exceeding 20' |  |  |  |  |  |  |
|  | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP | RCCP | CSP | ESCP | PVC | CPVC | PE | CPP |
| 10 | NA | 2 | X | X | X | X | X | NA | NA | 3 | X | X | X | X | NA |
| 12 | III | 2 | X | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 15 | III | 3 | X | X | X | NA | NA | X | IV | NA | NA | X | X | NA | X |
| 18 | III | NA | X | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 21 | III | NA | NA | X | X | NA | NA | NA | IV | NA | NA | X | X | NA | NA |
| 24 | III | NA | NA | X | X | X | NA | NA | IV | NA | NA | X | X | X | NA |
| 27 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 30 | III | NA | NA | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 33 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 36 | III | NA | NA | X | X | X | NA | NA | IV | NA | NA | X | X | X | NA |
| 42 | III | NA | NA | X | NA | X | NA | NA | IV | NA | NA | X | NA | X | NA |
| 48 | III | NA | NA | X | NA | X | NA | NA | IV | NA | NA | X | NA | X | NA |
| 54 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 60 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 66 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 72 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 78 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 84 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 90 | III | NA | NA | NA | NA | NA | NA | NA | 1680 | NA | NA | NA | NA | NA | NA |
| 96 | III | NA | NA | NA | NA | NA | NA | NA | 1690 | NA | NA | NA | NA | NA | NA |
| 102 | III | NA | NA | NA | NA | NA | NA | NA | 1700 | NA | NA | NA | NA | NA | NA |
| 108 | 1360 | NA | NA | NA | NA | NA | NA | NA | 1710 | NA | NA | NA | NA | NA | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

CSP	Concrete Sewer, Storm drain, and Culvert Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

PE	Polyethylene Pipe with a Smooth Interior

CPE	Corrugated Polyethylene Pipe with a Smooth Interior

CPP	Corrugated Polypropylene pipe with a Smooth Iinterior

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

*	May also use Standard Strength Clay Pipe

Note	RCCP with a number instead of a Roman numeral shall be furnished according to AASHTO M170 Section 6.  This number represents the D-load to produce a 0.01 in crack.

| STORM SEWERS (metric) KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 3 |  |  |  |  |  |  |  | Type 4 |  |  |  |  |  |  |
|  | Fill Height: Greater than 3 m not exceeding 4.5 m |  |  |  |  |  |  |  | Fill Height: Greater than 4.5 m not exceeding 6 m |  |  |  |  |  |  |
|  | RCCP | CSP | ESCP | PVC | CPVC | PE | CPE | CPP | RCCP | CSP | ESCP | PVC | CPVC | PE | CPP |
| 250 | NA | 2 | X | X | X | X | X | NA | NA | 3 | X | X | X | X | NA |
| 300 | III | 2 | X | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 375 | III | 3 | X | X | X | NA | NA | X | IV | NA | NA | X | X | NA | X |
| 450 | III | NA | X | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 525 | III | NA | NA | X | X | NA | NA | NA | IV | NA | NA | X | X | NA | NA |
| 600 | III | NA | NA | X | X | X | NA | NA | IV | NA | NA | X | X | X | NA |
| 675 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 750 | III | NA | NA | X | X | X | NA | X | IV | NA | NA | X | X | X | NA |
| 825 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 900 | III | NA | NA | X | X | X | NA | NA | IV | NA | NA | X | X | X | NA |
| 1050 | III | NA | NA | X | NA | X | NA | NA | IV | NA | NA | X | NA | X | NA |
| 1200 | III | NA | NA | X | NA | X | NA | NA | IV | NA | NA | X | NA | X | NA |
| 1350 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 1500 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 1650 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 1800 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 1950 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 2100 | III | NA | NA | NA | NA | NA | NA | NA | IV | NA | NA | NA | NA | NA | NA |
| 2250 | III | NA | NA | NA | NA | NA | NA | NA | 80 | NA | NA | NA | NA | NA | NA |
| 2400 | III | NA | NA | NA | NA | NA | NA | NA | 80 | NA | NA | NA | NA | NA | NA |
| 2550 | III | NA | NA | NA | NA | NA | NA | NA | 80 | NA | NA | NA | NA | NA | NA |
| 2700 | 70 | NA | NA | NA | NA | NA | NA | NA | 80 | NA | NA | NA | NA | NA | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

CSP	Concrete Sewer, Storm drain, and Culvert Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

PE	Polyethylene Pipe with a Smooth Interior

CPE	Corrugated Polyethylene Pipe with a Smooth Interior

CPP	Corrugated Polypropylene pipe with a Smooth Interior

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

*	May also use Standard Strength Clay Pipe

Note	RCCP with a number instead of a Roman numeral shall be furnished according to AASHTO M170 Section 6.  This number represents the metric D-load to produce a 25.4 micro-meter crack.

| STORM SEWERS KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 5 |  |  | Type 6 |  |  | Type 7 |  |
|  | Fill Height: Greater than 20' not exceeding 25' |  |  | Fill Height: Greater than 25' not exceeding 30' |  |  | Fill Height: Greater than 30' not exceeding 35' |  |
|  | RCCP | PVC | CPVC | RCCP | PVC | CPVC | RCCP | CPVC |
| 10 | NA | X | X | NA | X | X | NA | X |
| 12 | IV | X | X | V | X | X | V | X |
| 15 | IV | X | X | V | X | X | V | X |
| 18 | IV | X | X | V | X | X | V | X |
| 21 | IV | X | X | V | X | X | V | X |
| 24 | IV | X | X | V | X | X | V | X |
| 27 | IV | NA | NA | V | NA | NA | V | NA |
| 30 | IV | X | X | V | X | X | V | X |
| 33 | IV | NA | NA | V | NA | NA | V | NA |
| 36 | IV | X | X | V | X | X | V | X |
| 42 | IV | X | NA | V | X | NA | V | NA |
| 48 | IV | X | NA | V | X | NA | V | NA |
| 54 | IV | NA | NA | V | NA | NA | V | NA |
| 60 | IV | NA | NA | V | NA | NA | V | NA |
| 66 | IV | NA | NA | V | NA | NA | V | NA |
| 72 | V | NA | NA | V | NA | NA | V | NA |
| 78 | 2020 | NA | NA | 2370 | NA | NA | 2730 | NA |
| 84 | 2020 | NA | NA | 2380 | NA | NA | 2740 | NA |
| 90 | 2030 | NA | NA | 2390 | NA | NA | 2750 | NA |
| 96 | 2040 | NA | NA | 2400 | NA | NA | 2750 | NA |
| 102 | 2050 | NA | NA | 2410 | NA | NA | 2760 | NA |
| 108 | 2060 | NA | NA | 2410 | NA | NA | 2770 | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

Note	RCCP with a number instead of a Roman numeral shall be furnished according to AASHTO M170 Section 6.  This number represents the D-load to produce a 0.01 in crack.

| STORM SEWERS (metric) KIND OF MATERIAL PERMITTED AND STRENGTH REQUIRED FOR A GIVEN PIPE DIAMETERS AND FILL HEIGHTS OVER THE TOP OF THE PIPE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal Diameter in. | Type 5 |  |  | Type 6 |  |  | Type 7 |  |
|  | Fill Height: Greater than 20' not exceeding 25' |  |  | Fill Height: Greater than 25' not exceeding 30' |  |  | Fill Height: Greater than 30' not exceeding 35' |  |
|  | RCCP | PVC | CPVC | RCCP | PVC | CPVC | RCCP | CPVC |
| 250 | NA | X | X | NA | X | X | NA | X |
| 300 | IV | X | X | V | X | X | V | X |
| 375 | IV | X | X | V | X | X | V | X |
| 450 | IV | X | X | V | X | X | V | X |
| 525 | IV | X | X | V | X | X | V | X |
| 600 | IV | X | X | V | X | X | V | X |
| 675 | IV | NA | NA | V | NA | NA | V | NA |
| 750 | IV | X | X | V | X | X | V | X |
| 825 | IV | NA | NA | V | NA | NA | V | NA |
| 900 | IV | X | X | V | X | X | V | X |
| 1050 | IV | X | NA | V | X | NA | V | NA |
| 1200 | IV | X | NA | V | X | NA | V | NA |
| 1350 | IV | NA | NA | V | NA | NA | V | NA |
| 1500 | IV | NA | NA | V | NA | NA | V | NA |
| 1650 | IV | NA | NA | V | NA | NA | V | NA |
| 1800 | V | NA | NA | V | NA | NA | V | NA |
| 1950 | 100 | NA | NA | 110 | NA | NA | 130 | NA |
| 2100 | 100 | NA | NA | 110 | NA | NA | 130 | NA |
| 2250 | 100 | NA | NA | 110 | NA | NA | 130 | NA |
| 2400 | 100 | NA | NA | 120 | NA | NA | 130 | NA |
| 2550 | 100 | NA | NA | 120 | NA | NA | 130 | NA |
| 2700 | 100 | NA | NA | 120 | NA | NA | 130 | NA |

RCCP	Reinforced Concrete Culvert, Storm Drain, and Sewer Pipe

PVC	Polyvinyl Chloride Pipe

CPVC	Corrugated Polyvinyl Chloride Pipe

ESCP	Extra Strength Clay Pipe

X	This material may be used for the given pipe diameter and fill height.

NA 	This material is Not Acceptable for the given pipe diameter and fill height.

Note	RCCP with a number instead of a Roman numeral shall be furnished according to AASHTO M170 Section 6.  This number represents the metric D-load to produce a 25.4 micro-meter crack.

Revise the sixth paragraph of Article 550.06 of the Standard Specifications to read:

“PVC, PE and CPP pipes shall be joined according to the manufacturer’s specifications.”

Revise the first and second paragraphs of Article 550.08 of the Standard Specifications to read:

“	550.08	Deflection Testing for Storm Sewers.  All PVC, PE, and CPP storm sewers shall be tested for deflection not less than 30 days after the pipe is installed and the backfill compacted.  The testing shall be performed in the presence of the Engineer.

For PVC, PE, and CPP storm sewers with diameters 24 in. (600 mm) or smaller, a mandrel drag shall be used for deflection testing.  For PVC, PE, and CPP storm sewers with diameters over 24 in. (600 mm), deflection measurements other than by a mandrel shall be used.”

Revise the fifth paragraph of Article 550.08 to read as follows.

“The outside diameter of the mandrel shall be 95 percent of the base inside diameter.  For all PVC pipe the base inside diameter shall be defined using ASTM D 3034 methodology.  For all PE and CPP pipe, the base inside diameter shall be defined as the average inside diameter based on the minimum and maximum tolerances specified in the corresponding ASTM or AASHTO material specifications.”

Revise the first paragraph of Article 1040.03 of the Standard Specifications to read:

“	1040.03	Polyvinyl Chloride (PVC) Pipe.  Acceptance testing of PVC pipe and fittings shall be accomplished during the same construction season in which they are installed.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties.  The pipe shall meet the following additional requirements.”

Delete Articles 1040.03(e) and (f) of the Standard Specifications.

Revise Articles 1040.04(c) and (d) of the Standard Specifications to read:

“	(c)	PE Profile Wall Pipe for Insertion Lining.  The pipe shall be according to ASTM F 894.  When used for insertion lining of pipe culverts, the pipe liner shall have a minimum pipe stiffness of 46 psi (317 kPa) at five percent deflection for nominal inside diameters of 42 in. (1050 mm) or less.  For nominal inside diameters of greater than 42 in. (1050 mm), the pipe liner shall have a minimum pipe stiffness of 32.5 psi (225 kPa) at five percent deflection.  All sizes shall have wall construction that presents essentially smooth internal and external surfaces.

(d)	PE Pipe with a Smooth Interior.  The pipe shall be according to ASTM F 714 (DR 32.5) with a minimum cell classification of PE 335434 as defined in ASTM D 3350.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties and the resin used to manufacture the pipe meets or exceeds the minimum cell classification requirements.”

Add the following to Section 1040 of the Standard Specifications:

“	1040.08	Polypropylene (PP) Pipe.  Storage and handling shall be according to the manufacturer's recommendations, except in no case shall the pipe be exposed to direct sunlight for more than six months.  Acceptance testing of the pipe shall be accomplished during the same construction season in which it is installed.  The section properties shall be according to the manufacturer pre-submitted geometric properties on file with the Department.  The manufacturer shall submit written certification that the material meets those properties.  The pipe shall meet the following additional requirements.

(a)	Corrugated PP Pipe with a Smooth Interior.  The pipe shall be according to AAHSTO M 330 (nominal size – 12 to 60 in. (300 to 1500 mm)).  The pipe shall be Type S or D.

(b)	Perforated Corrugated PP Pipe with A Smooth Interior.  The pipe shall be according to AASHTO M 330 (nominal size – 12 to 60 in. (300 to 1500 mm)).  The pipe shall be Type SP.  In addition, the top centerline of the pipe shall be marked so that it is readily visible from the top of the trench before backfilling, and the upper ends of the slot perforations shall be a minimum of ten degrees below the horizontal.”

80325