All Regional Engineers

Omer M. Osman, P.E.

Special Provision for Paved Shoulder Removal

January 10, 2014

This special provision was developed by the Bureau of Design and Environment, at the request of Industry, to address the variability in thickness for paved shoulder removal.

This special provision should be inserted into contracts involving the removal of either HMA or PCC shoulders.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the April 25, 2014 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory 
January 10, 2014.

80337m

# Paved Shoulder Removal (bde)

Effective:  April 1, 2014

Revise the first paragraph of Article 440.07(b) of the Standard Specifications to read:

“	(b)	Measured Quantities.  Pavement removal, driveway pavement removal, and paved shoulder removal will be measured for payment in place and the area computed in square yards (square meters).”

Revise Article 440.07(c) of the Standard Specifications to read:

“	(c)	Adjustment of Quantities.  The quantity of pavement removal and paved shoulder removal will be adjusted if their respective thickness varies more than 15 percent from that shown on the plans.  The quantity will be either increased or decreased according to the following table.

| % change of thickness | % change of quantity |
| --- | --- |
| 0 to less than 15 15 to less than 20 20 to less than 30 30 to less than 50 | 0 10 15 20 |

If the thickness of the existing pavement varies by 50 percent or more from that shown on the plans, the character of the work will be considered significantly changed and an adjustment to the contract will be made according to Article 104.02.

When an adjustment is made for variations in pavement or shoulder thickness a resulting adjustment will also be made in the earthwork quantities when applicable.

No adjustment will be made for variations in the amount of reinforcement.”

80337