All Regional Engineers

Omer M. Osman, P.E.

Special Provision for Speed Display Trailer

April 18, 2014

This special provision was developed by the Bureau of Safety Engineering to enhance safety of the traveling public and workers in work zones by alerting drivers of their speed, thus deterring them from driving above the posted work zone speed limit.

This special provision should be inserted into all freeway and expressway projects involving Highway Standard 701400.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the August 1, 2014 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory April 18, 2014.

80340m

# Speed Display Trailer (bde)

Effective:  April 2, 2014

Add the following to Article 701.15(l) of the Standard Specifications:

“	(l)	Speed Display Trailer.  A speed display trailer shall be utilized on freeways and expressways as part of Highway Standard 701400.  The trailer shall be placed on the right hand side of the roadway adjacent to, or within 100 ft (30 m) beyond, the first work zone speed limit sign.

Whenever the speed display trailer is not in use, it shall be considered non-operating equipment and shall be stored according to Article 701.11.”

Add the following to Article 701.20 of the Standard Specifications:

“	(k)	Speed Display Trailer will be paid for at the contract unit price per calendar month or fraction thereof for each trailer as SPEED DISPLAY TRAILER.”

Add the following to Article 1106.02 of the Standard Specifications:

“	(o)	Speed Display Trailer.  The speed display trailer shall consist of a LED speed indicator display with self-contained, one-direction radar mounted on an orange see-through trailer.  The height of the display and radar shall be such that it will function and be visible when located behind concrete barrier.

The speed measurement shall be by radar and provide a minimum detection distance of 1000 ft (300 m).  The radar shall have an accuracy of ±1 mile per hour.

The speed indicator display shall face approaching traffic and shall have a sign legend of “YOUR SPEED” immediately above or below the speed display.  The digital speed display shall show two digits (00 to 99) in mph.  The color of the changeable message legend shall be a yellow legend on a black background.  The minimum height of the numerals shall be 18 in. (450 mm), and the nominal legibility distance shall be at least 750 ft (250 m).

The speed indicator display shall be equipped with a violation alert that flashes the displayed detected speed when the posted limit is exceeded.  The speed indicator shall have a maximum speed cutoff.  The display shall include automatic dimming for nighttime operation.

The speed indicator measurement and display functions shall be equipped with the power supply capable of providing 24 hours of uninterrupted service.”

80340