# SAND DRAINAGE BLANKET

Effective:  April 10, 1980					Revised:  November 1, 2006

The work of this item consists of furnishing all materials and equipment necessary for the construction of a sand drainage blanket to form a horizontal drainage layer between the proposed embankment and the natural ground.  Vertical plastic drainage wicks are to be constructed from this blanket.

Materials.  The sand for the drainage blanket shall conform to Section 1003 of the Standard Specifications.  The gradation shall be FA 6, except that the percentage passing the 75 micron (No. 200) sieve shall be 2 + 2.

Construction Requirements.  The sand drainage blanket shall be constructed to the thickness and within the lines and grades shown on the plans.  Sand may be placed by end dumping or other approved method, and spread uniformly over the site to the neat lines shown on the plans.  No compactive effort will be required on the sand drainage blanket.

Prior to placement of the embankment, the sand drainage blanket shall be reshaped to conform to the lines shown on the plans.

If the equipment used for construction of the vertical plastic drainage wicks cannot be supported directly on the sand drainage blanket without displacing the underlying soils, the Contractor may be permitted to place a small portion of the embankment material to be used as a working platform for installing the vertical sand drains as directed by the Engineer.

Method of Measurement.  The sand drainage blanket will be measured as cubic yards (cubic meters) of sand placed and no allowance will be made for any sand placed outside the lines as specified herein or as directed by the Engineer.

Basis of Payment.  The sand drainage blanket will be paid for as plan quantity at the contract unit price per cubic yard (cubic meter) for SAND DRAINAGE BLANKET.  No additional payment will be made for additional sand placed because of settlement.