# BOARD TEXTURE FINISH

Effective:  April 1979						Revised:  November 1, 2006

This item shall consist of all labor and materials required to provide a board texture finish to concrete surfaces as specified on plans, by the use of suitable form liners.

The texture pattern shall be wood grain, rough sawn cedar, random width and length board design similar to that shown on Sheet ______ of these Special Provisions.  The form liner used and the pattern shall be approved by the Engineer before commencing work.

All concrete shall be furnished and placed in accordance with the normal requirements of Sections 503 or 504 of the Standard Specifications as applicable, except as hereinafter specified for surfaces requiring a board texture finish.

The Contractor shall take special care to maintain the specified clearance of reinforcement bars from concrete surface of the texture.  The nominal dimension as shown on the plans should be taken as being from the deepest indentation of the board mark finish shown on the detail (next sheet).  Form ties shall be of the "snap tie" type with sufficient break-back so that holes can be plugged.

In order to establish procedures to achieve a finish satisfactory to the Engineer, the Contractor shall submit to the Engineer for approval a 2x2 foot (0.6 m x 0.6 m) sample panel prior to casting the project member or unit.  The sample panel shall be cast in a vertical position using the concrete mix and aggregate proposed for use in the work.  Concreting operations and stripping of forms in preparation of the sample panel shall follow actual work procedures insofar as is practical.  In any event the approved panel shall be used as the control for the appearance of the finished work and work unsatisfactory to the Engineer shall be corrected, or redone as required by the Engineer.

This work as herein specified shall not be measured for payment but shall be considered as included in the item for the concrete involved.