# STONE FACED FINISH

This item shall consist of all labor and materials required to provide a stone faced finish to concrete surfaces as specified on plans, by the use of suitable form liners.

The stone faced pattern shall be similar to that shown on Sheet 8 of these Special Provisions.  The form liner used and the pattern shall be approved by the Engineer before commencing work.

All concrete shall be furnished and placed in accordance with the normal requirements of Section 503 of the Standard Specifications as applicable, except as hereinafter specified for surfaces requiring a stone faced finish.

The Contractor shall take special care to maintain the specified clearance of reinforcement bars from concrete surface of the finish.  The nominal dimensions as shown on the plans should be taken as being from the deepest indentation of the stone faced finish shown on the detail on Sheet 8 of these Special Provisions.  Form ties shall be of the “snap tie” type with sufficient break-back so that holes can be plugged.

In order to establish procedures to achieve a finish satisfactory to the Engineer, the Contractor shall submit to the Engineer for approval a 0.6 m x 0.6 m (2x2 foot) sample panel prior to casting the project member or unit.  The sample panel shall be cast in a vertical position using the concrete mix and aggregate proposed for use in the work.  Concreting operations and stripping of forms in preparation of the sample panel shall follow actual work procedures insofar as is practical.  In any event the approved panel shall be used as the control for the appearance of the finished work and work unsatisfactory to the Engineer shall be corrected, or redone as required by the Engineer.

This work as herein specified shall not be measured for payment but shall be considered as incidental to the item for the concrete involved.