# TERMINAL FACILITY

Effective:  November 1, 1992					Revised:  November 1, 2006

Description

This item shall consist of furnishing and installing a loop terminal facility at the location indicated on the plans, or as directed by the Engineer.

Materials

The terminal facility shall be provided and wired as shown on the drawing SPECIAL DETAIL FOR TRAFFIC COUNTERS USING TERMINAL FACILITY.  The moisture-proof mounting box shall be mounted on a 4 in x 6 in x 6 ft (100 mm x 150 mm x 1.83 m) or 4 in x 4 in x 6 ft (100 mm x 100 mm x 1.83 m) treated wood post (as directed by the Engineer) meeting the requirements of Section 1007 of the Standard Specifications.  The treated wood post shall be installed at the location indicated on the plans or as directed by the Engineer.  The PVC conduit will be of the same type used in PVC CONDUIT IN TRENCH.  The elbow and reducing adapter required to mount the moisture-proof mounting box to the PVC conduit shall be installed in a manner that will provide a watertight seal.

Installation Details

The #18 3-pair shielded wire loop leads shall be placed in the box to provide slack as necessary to connect to the terminal strip.  The two lead wires from each loop shall be connected to their respective terminals on the terminal strip.

The moisture-proof mounting box shall be fastened to the treated wood post through at least two lugs with 1-1/2 in (38 mm) #8 round head wood screws.  The moisture-proof mounting type box shall be attached with a reducing adapter to the vertical section of PVC. conduit.  The PVC conduit shall be held in place against the treated wood post with a clamp located approximately 9 in (225 mm) above the ground line.

Care should be taken during the backfilling of the trench to not put excessive pressure on the elbow and cause a separation of conduit and elbow.

Basis of Payment

This work will be paid for at the contract unit price each for TERMINAL FACILITY, of the type specified and as shown on Special Detail.