Regional Engineers

Maureen M. Addis

Special Provision for Pavement Marking Removal

April 15, 2016

This special provision was developed by the Bureau of Operations to eliminate shot blasting as a pavement marking removal option and creates separate grinding and water blasting pavement marking removal pay items.

Check Sheet #27 Pavement Marking Removal should no longer be used with the creation of separate pay items for grinding and water blasting.

This special provision should be inserted into contracts requiring pavement marking removal.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the July 29, 2016 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory April 15, 2016.

80371m

# pavement marking removal (bde)

Effective:  July 1, 2016

Revise Article 783.02 of the Standard Specifications to read:

“	783.02	Equipment.  Equipment shall be according to the following.

Item	Article/Section

(a)	Grinders (Note 1)

(b)	Water Blaster with Vacuum Recovery 	1101.12

Note 1.  Grinding equipment shall be approved by the Engineer.”

Revise the first paragraph of Article 783.03 of the Standard Specifications to read:

“	783.03	Removal of Conflicting Markings.  Existing pavement markings that conflict with revised traffic patterns shall be removed.  If darkness or inclement weather prohibits the removal operations, such operations shall be resumed the next morning or when weather permits.  In the event of removal equipment failure, such equipment shall be repaired, replaced, or leased so removal operations can be resumed within 24 hours.”

Revise the first and second sentences of the first paragraph of Article 783.03(a) of the Standard Specifications to read:

“	The existing pavement markings shall be removed by the method specified and in a manner that does not materially damage the surface or texture of the pavement or surfacing.  Small particles of tightly adhering existing markings may remain in place, if in the opinion of the Engineer, complete removal of the small particles will result in pavement surface damage.”

Revise the first paragraph of Article 783.04 of the Standard Specifications to read:

“	783.04	Cleaning.  The roadway surface shall be cleaned of debris or any other deleterious material by the use of compressed air or water blast.”

Revise the first paragraph of Article 783.06 of the Standard Specifications to read:

“	783.06	Basis of Payment.  This work will be paid for at the contract unit price per each for RAISED REFLECTIVE PAVEMENT MARKER REMOVAL, or at the contract unit price per square foot (square meter) for PAVEMENT MARKING REMOVAL – GRINDING and/or PAVEMENT MARKING REMOVAL – WATER BLASTING.”

Delete Article 1101.13 from the Standard Specifications.

80371