Regional Engineers

Maureen M. Addis

Special Provision for Equipment Parking and Storage

August 4, 2017

This special provision was developed by the Bureau of Safety Programs and Engineering to specify the minimum clear distance for equipment parking and storage behind temporary concrete barrier to allow for the expected deflection after an impact and increase the safety of road users and workers.

This special provision should be inserted into all contracts.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 17, 2017 and subsequent lettings.  The Project Development and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory August 4, 2017.

80388m

# Equipment Parking and Storage (bde)

Effective:  November 1, 2017

Replace the first paragraph of Article 701.11 of the Standard Specifications with the following.

“	701.11	Equipment Parking and Storage.  During working hours, all vehicles and/or nonoperating equipment which are parked, two hours or less, shall be parked at least 8 ft (2.5 m) from the open traffic lane.  For other periods of time during working and for all nonworking hours, all vehicles, materials, and equipment shall be parked or stored as follows.

(a)	When the project has adequate right-of-way, vehicles, materials, and equipment shall be located a minimum of 30 ft (9 m) from the pavement.

(b)	When adequate right-of-way does not exist, vehicles, materials, and equipment shall be located a minimum of 15 ft (4.5 m) from the edge of any pavement open to traffic.

(c)	Behind temporary concrete barrier, vehicles, materials, and equipment shall be located a minimum of 24 in. (600 mm) behind free standing barrier or a minimum of 6 in. (150 mm) behind barrier that is either pinned or restrained according to Article 704.04.  The 24 in. or 6 in. measurement shall be from the base of the non-traffic side of the barrier.

(d)	Behind other man-made or natural barriers meeting the approval of the Engineer.”

80388