Regional Engineers

Jack A. Elston

Special Provision for Traffic Barrier Terminal, Type 1 Special

July 27, 2018

This special provision was developed by the Bureau of Design and Environment to remove reference to any particular crash testing standard and to revise the pay limit of a Type 1 Special terminal to match the product’s length as shown on the manufacturer’s drawings.

This special provision should be inserted into all contracts requiring the installation of a traffic barrier terminal, Type 1 Special.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 9, 2018 and subsequent lettings.  The Project Coordination and Implementation Section will include a copy in the contract.

This special provision will be available on the transfer directory July 27, 2018.

80403m

# TRAFFIC BARRIER TERMINAL, TYPE 1 SPECIAL (bde)

Effective:  November 1, 2018

Revise Article 631.04 of the Supplemental Specifications to read:

“	631.04	Traffic Barrier Terminal, Type 1 Special (Tangent) and Traffic Barrier Terminal, Type 1 Special (Flared).  These terminals shall be on the Department’s qualified product list.

The terminal shall be installed according to the manufacturer’s specifications.  The beginning length of need point of the terminal shall be placed within 12 ft 6 in (3.8 m) of the length of need point shown on the plans.

The terminal shall be delineated with a terminal marker direct applied.  No other guardrail delineation shall be attached to the terminal section.”

Revise the first paragraph of Article 631.12 of the Standard Specifications to read:

“	631.12	Method of Measurement.  The various types of traffic barrier terminals will be measured for payment, complete in place, in units of each.  The pay limit between the traffic barrier terminal and the adjacent guardrail shall be as shown on the plans, except for the following:

- Traffic Barrier Type 1, Special.  The pay limit for a traffic barrier, Type 1 special shall be as shown on the manufacturer’s drawing(s).

(b)	Traffic Barrier Type 10.  The pay limit for the traffic barrier terminal, Type 10 shall be at the centerline of the end shoe splice.”

80403