## BORROW EXCAVATION

Revised 08/02/21

In addition to the requirements of Article 107.22 and section 204, the contractor shall submit to the department a request for approval of the material in a proposed borrow pit.  The submittal shall contain:

(a) An 8 1/2" x 11" topographic map or sketch containing the dimensions of the area proposed and the locations of pertinent landmarks, the name(s) of the property owner, and the proposed depth of cut.  Copies of this map may also be used for subsequent submittal required for the archeological survey of the borrow pit.

The contractor shall provide access for truck mounted drilling equipment, if required, to and from and in all areas where he/she requests material investigations.

Borrow excavation which is to be used in the roadway embankment without restrictions must have more than 35% of the total sample passing the No. 200 sieve.  The soil must have a liquid limit value of 50.0 or less and a plasticity index value of 12.0 or more as defined by the AASHTO Classification System.  These soils shall be capable of obtaining the required design embankment strengths.  Soils proposed by the contractor for furnished excavation or borrow which do not meet the aforementioned requirements will be assigned varying degrees of restrictions up to and including complete rejection depending on the nature and engineering properties of the material.  These restrictions, if any, will be set forth in the proposed borrow material report.

It is anticipated that, depending upon the workload at the time, this field and laboratory investigation may take up to 15 days.

DESIGNER NOTES:

Use:  All projects with borrow excavation.