## HOT-MIX ASPHALT REMOVAL SPECIAL

Effective 1/2/97  Revised 7/27/21

The work shall consist of the removal and disposal of the existing HMA surface and waterproofing membrane system from the bridge deck as shown on the plans, in accordance with the applicable portions of Articles 440.04 and 440.06, except milling equipment will not be allowed, and as directed by the Engineer.  The removal shall be done in such a manner that the concrete deck is not damaged. Removal of HMA surface by the use of radiant or direct heat will not be permitted.

This work, as herein specified, will be paid for at the contract unit price per square meter (square yard) for HOT-MIX ASPHALT SURFACE REMOVAL SPECIAL, which price shall include removal of all HMA surface, and waterproofing membrane system.

DESIGNER NOTES:

Use:  All PPC deck beam bridges where removal is down to the bare deck.