## PAVED CONCRETE DITCH BREAKING

Revised 12/11/19

This item consists of breaking up the existing paved concrete ditch, excavating the side slopes, furnishing, transporting, and placing stone riprap.  This work shall be performed in accordance with the applicable portions of sections 281 and 1005 of the Standard Specifications, as shown in the plans, and as specified herein.

The existing paved concrete ditch shall be broken in place, at locations shown in the schedule of work, so that the broken concrete pieces shall not exceed 12" in the largest dimension.  The broken concrete shall remain in place.

The earth side slopes adjacent to the paved ditch shall be excavated so as to allow placement of the riprap.  The excavated material shall be disposed of in accordance with Article 281.05.

The stone riprap shall be A3 and shall be placed over and adjacent to the existing broken concrete as shown in the plans.  The finished side slopes of the stone riprap ditch shall match the existing grade.

This work will be paid for at the contract unit price per ton for STONE RIPRAP DITCH, and shall include the price of breaking the existing paved concrete ditch, excavating the side slopes, and furnishing, transporting, and placing stone riprap.