## RRPM protection

Revised 1/11/2018

This work shall consist of temporarily covering raised reflective pavement markers to protect them from microsurfacing operations and removal of the protective covering.

Existing raised reflective pavement markers (RRPM) shall be temporarily covered with an adhesive tape product.  The adhesive tape shall be either pavement marking tape or an adhesive tape product made of a fabric laminated to a backing of low density polyethylene, similar to duct tape.  The tape shall be at least 1 in. (25 mm) larger in all dimensions than the width of the RRPM being covered.  The covering of existing RRPMs shall be scheduled immediately prior to the microsurfacing operations.

As soon as practical after the microsurfacing operation is completed the protective covering shall be removed from the RRPMs.

In the event the adhesive product fails to provide adequate protection for the RRPMs, the Contractor shall replace the damaged RRPM at his own expense.

This work shall be measured in place per each raised reflective pavement marker that is covered.

## This work shall be paid per each for RRPM PROTECTION