## STRINGLINE GRADE CONTROL

Effective 1990     Revised 2/10/17

The Contractor shall use stringline grade control or an electronic grade control system that has been approved by the Engineer for the ______________________________

The stringline grade supports shall be placed at 25' intervals. The stringline and supports shall be offset so as not to interfere with rolling operations.

A tolerance of ½" from the proposed grade elevations will be permitted for the final earth subgrade; a tolerance of ¼" will be permitted for the final lime modified soil subgrade.

It is understood that the cost of complying with these requirements is included in the unit prices of the various pay items involved, and no additional compensation will be allowed.

DESIGNER NOTES:

SUGGESTED USES:

1.	final shaping of the earth subgrade

2.	final trimming of the lime modified soil subgrade

3.	laydown of the first lift of bituminous binder course

4.	laydown of each lift of bituminous concrete binder course

5.	laydown of the surface course

6.   at approaches and departures to railroad tracks

Consult with Construction.