## TEMPORARY TRAFFIC SIGNAL INSTALLATION

Revised 12/11/19

The temporary traffic signal/lighting installation will be in accordance with section 890 and this special provision.  The temporary traffic signal/lighting system will not be shown on the plans.  The Contractor will be required to present for approval the design of the temporary traffic signal/lighting system and modifications for each stage of construction.  The Contractor will be required to keep the existing signal/lighting, the temporary signal/lighting, the new signal/lighting, or a combination of the above signal/lighting systems including pedestrian signals (if existing) operational during all phases of construction.  The Contractor shall present and have approved the proposed temporary traffic signal/lighting installation before any changes are made to the existing signal/lighting installation.  Any Span Wire Mounted Traffic Signals (if necessary) shall use the three-wire method of mounting.  The Contractor shall provide traffic signal faces, wood poles, span wire, temporary traffic signal cable, anchor devices, highway lighting luminaries, and all other materials required for the temporary installation.  The Contractor's temporary traffic signal installation shall meet the requirements of Chapter IV of the Manual on Uniform Traffic Control Devices at all times.  The Traffic Control Deficiency Deduction shall be applied when vehicle detection is not maintained to the greatest extent possible  The methods used to accomplish these tasks shall be as determined by the Contractor and approved by the District's Traffic Operations Engineer.

The Contractor shall install and place in operation each temporary traffic signal/lighting system.  During changes in the traffic signal system, the Contractor will be required to keep at least one traffic signal face flashing Red, with temporary stop signs in place, for each approach to the intersection.  The traffic signal shut downs (flashing all red operation) shall be kept to a minimum as required by the District's Traffic Operations Engineer.  All preparatory work for the change in the signal system shall be completed before the signal system is shut down.

The Contractor shall maintain each of these signal/lighting systems until all construction of this project is completed to the satisfaction of the Engineer.

This work shall be paid for at the contract unit price each, per intersection, for TEMPORARY TRAFFIC SIGNAL INSTALLATION.

DESIGNER NOTES:

Use:  Intersection projects with temporary signals.