## TRAFFIC CONTROL PLAN

Effective 1985     Revised 4/17/23

During the entire construction period, the road shall be kept open to traffic as follows:

(a)	The highway shall be kept open to at least one lane of traffic at all times, and to two lanes of traffic to the greatest extent possible.

(b)	Access to all public roads and private entrances shall be maintained during all stages of the work.

(c)	On the existing pavement during construction of the detour road and temporary structure.

(d)	On the detour road during construction of the proposed structure, grade change or relocation.

(e)	Over one lane of the existing or new bridge during construction of the opposite lane of the structure.

Prior to allowing traffic on any portion of the roadway that has been cold milled, the Contractor shall have erected "Rough Grooved Surface" and "Uneven Pavement" signs that conform to the details shown in the plans. A minimum of one sign at each end of the improvement will be required. The Contractor shall maintain the "Rough Grooved Surface" signs until the cold milled surface is covered with binder. The Contractor shall maintain the "Uneven Pavement" signs until the resurfacing operations are completed.  The cost of furnishing, erecting, maintaining, and removing the required signs shall be incidental to the contract.

The temporary rumble strips shown on the Traffic Control Detail in the plans will be required on this project.

At all locations where the Contractor's equipment is required to cross the traffic lanes, traffic control and protection in accordance with Standard 701306 shall be used. The "One Lane Road Ahead" signs indicated on this standard shall be replaced with "Road Machinery Ahead" signs W21-3(0)-48.  The cost of furnishing, erecting, maintaining, and removing the required signs shall be incidental to the contract.  Designer’s note: only use this paragraph when earth moving operations are involved such as 3R projects.