Regional Engineers

Jack A. Elston

Special Provision for Short Term and Temporary Pavement Markings

July 24, 2026

This special provision was developed by the Central Bureau of Operations to restrict the use of Type I pavement marking tape to applications of 14 days or fewer, and to discontinue the use of Type III tape as the Type IV material has greater wet reflectivity at a comparable price to the Type III material.  It has been revised to increase the width of short term and temporary pavement markings to 6 inches to provide additional guidance to the traveling public when navigating work zones.

This special provision should be inserted into contracts with short term or temporary pavement markings.

The districts should include the BDE Check Sheet marked with the applicable special provisions for the November 6, 2026 and subsequent lettings.  The Project Coordination and Implementation Section will include a copy in the contract.

80457m

# Short Term and Temporary PAvement Markings (bde)

Effective:  April 1, 2024

Revised:  November 1, 2026

Revise Article 701.02(d) of the Standard Specifications to read:

“(d)	Pavement Marking Tapes (Note 3) 	1095.06”

Add the following Note to the end of Article 701.02 of the Standard Specifications:

“	Note 3.  White or yellow pavement marking tape that is to remain in place longer than 14 days shall be Type IV tape.”

Revise Article 703.02(c) of the Standard Specifications to read:

“(c)	Pavement Marking Tapes (Note 1) 	1095.06”

Add the following Note to the end of Article 703.02 of the Standard Specifications:

“	Note 1.  White or yellow pavement marking tape that is to remain in place longer than 14 days shall be Type IV tape.”

Revise the first paragraph of Article 703.04 of the Standard Specifications to read:

“	703.04	Short Term Pavement Markings.  Before the lane is opened to traffic, short term pavement markings shall be installed between all lanes open to traffic.  Center line or lane line markings shall consist of an abbreviated pattern of single stripes 4 ft (1.2 m) in length and 6 in. (150 mm) wide at a maximum spacing of 40 ft (12 m) between stripes.  Center lines on two-lane highways shall be yellow and lane lines separating two or more lanes of traffic moving in the same direction shall be white.  Edge line markings shall be applied on multilane divided highways and other highways with a paved shoulder greater than 4 ft (1.2 m) wide.  Edge line markings shall consist of stripes 4 ft (1.2 m) in length and 6 in. (150 mm) wide at intervals of 50 ft (15 m) on ramps and 200 ft (60 m) on mainline installed at approximately a 45 degree diagonal pointing in the direction of traffic.  Short term pavement markings on the final wearing surface shall be transversely offset from the permanent pavement marking location as directed by the Engineer.  Short term pavement markings shall be removed within five calendar days after permanent pavement markings are installed.”

Revise the first paragraph of Article 703.05 of the Standard Specifications to read:

“	703.05	Temporary Pavement Marking.  When an intermediate course cannot be overlaid or if the final surface cannot be permanently marked within the time restrictions listed above, the full standard markings shall be installed with temporary pavement markings.   Temporary lane line, edge line, and center line markings shall be a minimum of 6 in. (150 mm) wide and of the same color as shown on the plans for the permanent markings. Other temporary markings shall be of the same color and dimensions as shown on the plans for the permanent markings.”

Revise Article 1095.06 of the Standard Specifications to read:

“1095.06	Pavement Marking Tapes.  Type I white or yellow marking tape shall consist of glass spheres embedded into a binder on a foil backing that is precoated with a pressure sensitive adhesive.  The spheres shall be of uniform gradation and distributed evenly over the surface of the tape.

Type IV tape shall consist of white or yellow tape with wet reflective media incorporated to provide immediate and continuing retroreflection in wet and dry conditions.  The wet retroreflective media shall be bonded to a durable polyurethane surface.  The patterned surface shall have approximately 40 ± 10 percent of the surface area raised and presenting a near vertical face to traffic from any direction.  The channels between the raised areas shall be substantially free of exposed reflective elements or particles.

Blackout tape shall consist of a matte black, non-reflective, patterned surface that is precoated with a pressure sensitive adhesive.

(a)	Color.  The white and yellow markings shall meet the following requirements for daylight reflectance and color, when tested, using a color spectrophotometer with 45 degrees circumferential/zero degree geometry, illuminant D65, and two degree observer angle.  The color instrument shall measure the visible spectrum from 380 to 720 nm with a wavelength measurement interval and spectral bandpass of 10 nm.

| Color | Daylight Reflectance %Y |
| --- | --- |
| White | 65 min. |
| Yellow * | 36 - 59 |

*Shall match Aerospace Material Specification Standard 595 33538 (Orange Yellow) and the chromaticity limits as follows.

| x | 0.490 | 0.475 | 0.485 | 0.530 |
| --- | --- | --- | --- | --- |
| y | 0.470 | 0.438 | 0.425 | 0.456 |

(b)	Retroreflectivity.  The white and yellow markings shall be retroreflective.  Reflective values measured in accordance with the photometric testing procedure of ASTM D 4061 shall not be less than those listed in the table below.  The coefficient of retroreflected luminance, RL, shall be expressed as average millicandelas/footcandle/sq ft (millicandelas/lux/sq m), measured on a 3.0 x 0.5 ft (900 mm x 150 mm) panel at 86 degree entrance angle.

| Coefficient of Retroreflected Luminance, RL, Dry |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type I |  |  | Type IV |  |  |
| Observation Angle | White | Yellow | Observation Angle | White | Yellow |
| 0.2° | 2700 | 2400 | 0.2° | 1300 | 1200 |
| 0.5° | 2250 | 2000 | 0.5° | 1100 | 1000 |

Wet retroreflectance shall be measured for Type IV under wet conditions according to ASTM E 2177 and meet the following.

| Wet Retroreflectance, Initial RL |  |
| --- | --- |
| Color | RL 1.05/88.76 |
| White | 300 |
| Yellow | 200 |

(c)	Skid Resistance. The surface of Type IV and blackout markings shall provide a minimum skid resistance of 45 BPN when tested according to ASTM E 303.

(d)	Application.  The pavement marking tape shall have a precoated pressure sensitive adhesive and shall require no activation procedures.  Test pieces of the tape shall be applied according to the manufacturer's instructions and tested according to ASTM D 1000, Method A, except that a stiff, short bristle roller brush and heavy hand pressure will be substituted for the weighted rubber roller in applying the test pieces to the metal test panel.  Material tested as directed above shall show a minimum adhesion value of 750 g/in. (30 g/mm) width at the temperatures specified in ASTM D 1000.  The adhesive shall be resistant to oils, acids, solvents, and water, and shall not leave objectionable stains or residue after removal.  The material shall be flexible and conformable to the texture of the pavement.

(e)	Durability.  Type IV and blackout tape shall be capable of performing for the duration of a normal construction season and shall then be capable of being removed intact or in large sections at pavement temperatures above 40 °F (4 °C) either manually or with a roll-up device without the use of sandblasting, solvents, or grinding.  The Contractor shall provide a manufacturer’s certification that the material meets the requirements for being removed after the following minimum traffic exposure based on transverse test decks with rolling traffic.

(1)	Time in place - 400 days

(2)	ADT per lane - 9,000 (28 percent trucks)

(3)	Axle hits - 10,000,000 minimum

Samples of the material applied to standard specimen plates will be measured for thickness and tested for durability in accordance with ASTM D 4060, using a CS-17 wheel and 1000-gram load, and shall meet the following criteria showing no significant change in color after being tested for the number of cycles indicated.

| Test | Type I | Type IV | Blackout |
| --- | --- | --- | --- |
| Minimum Initial Thickness, mils (mm) | 20 (0.51) | 65 (1.65) 1/ 20 (0.51) 2/ | 65 (1.65) 1/ 20 (0.51) 2/ |
| Durability (cycles) | 5,000 | 1,500 | 1,500 |

1/	Measured at the thickest point of the patterned surface.

2/	Measured at the thinnest point of the patterned surface.

The pavement marking tape, when applied according to the manufacturer's recommended procedures, shall be weather resistant and shall show no appreciable fading, lifting, or shrinkage during the useful life of the marking.  The tape, as applied, shall be of good appearance, free of cracks, and edges shall be true, straight, and unbroken.

(f)	Sampling and Inspection.

(1)	Sample.  Prior to approval and use of Type IV pavement marking tape, the manufacturer shall submit a notarized certification from an independent laboratory, together with the results of all tests, stating that the material meets the requirements as set forth herein.  The independent laboratory test report shall state the lot tested, the manufacturer’s name, and the date of manufacture.

After initial approval by the Department, samples and certification by the manufacturer shall be submitted for each subsequent batch of Type IV tape used.  The manufacturer shall submit a certification stating that the material meets the requirements as set forth herein and is essentially identical to the material sent for qualification.  The certification shall state the lot tested, the manufacturer’s name, and the date of manufacture.

(2)	Inspection.  The Contractor shall provide a manufacturer’s certification to the Engineer stating the material meets all requirements of this specification.  All material samples for acceptance tests shall be taken or witnessed by a representative of the Bureau of Materials and shall be submitted to the Engineer of Materials, 126 East Ash Street, Springfield, Illinois 62704-4766 at least 30 days in advance of the pavement marking operations.”

80457